Open System for Earthquake Engineering Simulation (OpenSees)

Command Language Manual

Silvia Mazzoni, Frank McKenna, Michael H. Scott, Gregory L. Fenves, Boris Jeremic

Contents

Introduction	7

Notation	7

Copyright	9

Tcl Basics	9

Tcl Commands	10

OpenSees Interpreter	13

Open System for Earthquake Engineering Simulation	14

ModelBuilder Object	15

Domain Object	16

Analysis Object	17

Recorder Object	18

Model Commands	19

Model Command	19

Basic Model Builder	19

build Command	20

Node Command	20

Mass Command	21

Constraints Commands	22

Single-Point Constraints	22

Multi-Point Constraints	24

uniaxialMaterial Command	27

Elastic Material	27

Elastic-Perfectly Plastic Material	28

Elastic-Perfectly Plastic Gap Material	30

Parallel Material	31

Series Material	32

Hardening Material	34

Steel01 Material	35

Concrete01 Material	36

Elastic-No Tension Material	37

Hysteretic Material	38

Viscous Material	39

Fedeas Materials	40

nDMaterial Command	45

Elastic Isotropic Material	45

J2 Plasticity Material	45

Plane Stress Material	46

Plate Fiber Material	46

Template Elasto-Plastic Material	47

section Command	51

Elastic Section	52

Uniaxial Section	53

Fiber Section	54

Section Aggregator	62

Elastic Membrane Plate Section	66

Plate Fiber Section	66

Bidirectional Section	66

element Command	68

Truss Element	68

Corotational Truss Element	69

Elastic Beam Column Element	70

NonLinear Beam-Column Elements	71

Zero-Length Elements	74

Quadrilateral Elements	75

Brick Elements	78

block Command	85

block2D Command	86

block3D Command	87

region Command	89

Geometric Transformation Command	90

Linear Transformation	90

P-Delta Transformation	92

Corotational Transformation	93

recorder Command	95

Node Recorder	95

EnvelopeNode Recorder	96

MaxNodeDisp Recorder	97

Drift Recorder	98

Element Recorder	98

EnvelopeElement Recorder	100

Display Recorder	101

Plot Recorder	102

playback Command	102

Analysis Commands	104

Time Series	105

Constant Time Series	106

Linear Time Series	106

Rectangular Time Series	107

Sine Time Series	108

Path Time Series	109

pattern Command	110

plain Pattern	110

UniformExcitation Pattern	112

MultipleSupport Pattern	113

analysis Command	116

Static Analysis	116

Transient Analysis	117

VariableTransient Analysis	117

constraints Command	119

Plain Constraints	120

Penalty Method	120

Lagrange Multipliers	121

Transformation Method	121

integrator Command	122

Load Control	123

Displacement Control	124

Minimum Unbalanced Displacement Norm	125

Arc-Length Control	125

Newmark Method	126

Hilbert-Hughes-Taylor	127

algorithm Command	128

Linear Algorithm	128

Newton Algorithm	128

Newton with Line Search Algorithm	129

Modified Newton Algorithm	129

Krylov-Newton Algorithm	130

BFGS Algorithm	130

Broyden Algorithm	130

test Command	131

Norm Unbalance Test	131

Norm Displacement Increment T	132

Energy Increment Test	133

numberer Command	134

Plain Numberer	134

RCM Numberer	135

system Command	136

BandGeneral SOE	137

BandSPD SOE	137

ProfileSPD SOE	137

SparseGeneral SOE	137

UmfPack SOE	137

SparseSPD SOE	138

analyze Command	139

eigen Command	140

dataBase Commands	141

FileDatastore Command	141

Miscellaneous Commands	143

print Command	143

reset Command	144

wipe Command	144

wipeAnalysis Command	144

loadConst Command	145

getTime Command	145

nodeDisp Command	145

video Command	146

play Command	146

How To....	147

...Run OpenSees	148

...Define Units & Constants	151

...Generate Matlab Commands	152

...Define Tcl Procedure	152

...Read External files	154

Building The Model	155

...Define Variables and Parameters	155

...Build Model and Define Nodes	157

...Build Model and Define Nodes using Variables	158

...Define Materials	159

...Define Elements	160

Defining Output	161

...Define Analysis-Output Generation	161

...Define Data-Plot During Analysis	162

Gravity Loads	162

...Define Gravity Loads	162

...Run Gravity Analysis	163

Static Analysis	163

...Define Static Pushover Analysis	163

...Run Static Pushover Analysis	164

Dynamic Analysis	165

...Define Dynamic Ground-Motion Analysis	165

...Run Dynamic Ground-Motion Analysis	166

...Combine Input-File Components	166

...Run Parameter Study	167

...Run Moment-Curvature Analysis on Section	168

...Determine Natural Period & Frequency	170

Index	173

This document is intended to outline the basic commands currently available with the OpenSees interpreter. This interpreter is an extension of the Tcl/Tk language for use with OpenSees.

Chapter 1

# Introduction

OpenSees is an object-oriented framework for finite element analysis. OpenSees' intended users are in the research community. A key feature of OpenSees is the interchangeability of components and the ability to integrate existing libraries and new components into the framework (not just new element classes) without the need to change the existing code. Core components, that is the abstract base classes, define the minimal interface (minimal to make adding new component classes easier but large enough to ensure all that is required can be accommodated).

- In This Chapter Notation 7 Copyright 9 Tcl Basics 9 OpenSees Interpreter 12

## Notation

The notation presented in this chapter is used throughout this document.

Input values are a string, unless the first character is a $, in which case an integer, floating point number or variable is to be provided. In the Tcl language, variable references start with the $ character. Tcl expressions can also be used as input to the commands where the input value is specified by the first character being a $.

Optional values are identified in enclosing <> braces.

When specifying a variable quantity of values, the command line contains (x $values). The number of values required, x, and the types of values, $values, are specified in the description of the command.

An arbitrary number of input values is indicated with the dot-dot-dot notation, i.e. $value1 $value2 ...

The OpenSees interpreter constructs objects in the order they are specified by the user. New objects are often based on previously-defined objects. When specified as an object parameter, a previously-defined object must have already been added to the Domain. This requirement is specified in the description of the command arguments.

Example command:

node $nodeTag (ndm $coords) <-mass (ndf $MassValues)>

This line executes the node command (page 18) assigns coordinates and masses to a specified node. The $nodeTag argument is an integer tag identifying the node.The coordinate arguments are specified with the parentheses () because the number of arguments is dependent on the definition of the model (ndm (page 17)): two arguments in 2D and three in 3D.

The mass specification at the node definition is optional. Therefore, it is enclosed in <> braces. The number of mass arguments is also dependent on the model definition, depending on the number of degrees of freedom assigned to a node (ndf (page 17)).

## Copyright

Copyright © 1999,2000 The Regents of the University of California. All Rights Reserved.

Permission to use, copy, modify, and distribute this software and its documentation for  educational, research and non-profit purposes, without fee, and without a written  agreement is hereby granted, provided that the above copyright notice, this paragraph  and the following three paragraphs appear in all copies.

Permission to incorporate this software into commercial products may be obtained by  contacting the University of California. [Bill Hoskins Office of Technology Licensing,  2150 Shattuck Avenue #150 Berkeley, CA 94720-1620, (510) 643-7201]

This software program and documentation are copyrighted by The Regents of the  University of California. The software program and documentation are supplied "as is",  without any accompanying services from The Regents. The Regents does not warrant  that the operation of the program will be uninterrupted or error-free. The end-user  understands that the program was developed for research purposes and is advised not  to rely exclusively on the program for any reason.

IN NO EVENT SHALL THE UNIVERSITY OF CALIFORNIA BE LIABLE TO  ANY PARTY FOR DIRECT, INDIRECT, SPECIAL, INCIDENTAL, OR  CONSEQUENTIAL DAMAGES, INCLUDING LOST PROFITS, ARISING OUT  OF THE USE OF THIS SOFTWARE AND ITS DOCUMENTATION, EVEN IF  THE UNIVERSITY OF CALIFORNIA HAS BEEN ADVISED OF THE  POSSIBILITY OF SUCH DAMAGE. THE UNIVERSITY OF CALIFORNIA  SPECIFICALLY DISCLAIMS ANY WARRANTIES, INCLUDING, BUT NOT  LIMITED TO, THE IMPLIED WARRANTIES OF MERCHANTABILITY AND  FITNESS FOR A PARTICULAR PURPOSE. THE SOFTWARE PROVIDED  HEREUNDER IS ON AN "AS IS" BASIS, AND THE UNIVERSITY OF  CALIFORNIA HAS NO OBLIGATIONS TO PROVIDE MAINTENANCE,  SUPPORT, UPDATES, ENHANCEMENTS, OR MODIFICATIONS.

## Tcl Basics

Tcl is a string-based procedural command language which allows the following:

- Variables and variable substitution

- Mathematical-expression evaluation

- Basic control structures (if , while, for, foreach)

- Procedures

- File manipulation

Tcl commands can be found at its web site:  Tcl/Tk Primer (see http://dev.scriptics.com/scripting/primer.html")

- Help

Tcl/Tk Primer (see http://dev.scriptics.com/scripting/primer.html") (http://dev.scriptics.com/scripting/primer.html)

Practical Programming in Tcl and Tk, Brent B. Welch, Prentice Hall.

### Tcl Commands

Tcl scripts are made up of commands separated by new lines or ;

The basic syntax for a Tcl command is:

command $arg1 $arg2  ...

| command | name of the Tcl command or user-defined procedure |
| --- | --- |
| $arg1 $arg2 ... | arguments for the command |

Tcl allows any argument to be nested command:

command [nested-command1] [nested-command2]

where the [] are used to delimit the nested commands. The Tcl interpreter will first evaluate the nested commands and will then evaluate the outer command with the results to the nested commands.

The most basic command in Tcl is the set command:

set $variable $value

bla bla bla, this needs to be more clear......

Take material from the scriptics page

Include the following commands:

proc

expr

file-management commands

for loops commands

if commands

#### Example Tcl Commands

| arithmetic | procedure | for & foreach functions |
| --- | --- | --- |
| >set a 1 1 >set b a a >set b $a 1 >expr 2 + 3 5 >expr 2 + $a 3 >set b [expr 2 + $a] 3 > | >proc sum {a b} { return [expr $a + $b] } >sum 2 3 5 >set c [sum 2 3] 5 > | for {set i 1} {$i < 10} {incr i 1} { puts “i equals $i” } set sum 0 foreach value {1 2 3 4} { set sum [expr $sum + $value] } puts $sum 10 > |
| file manipulation | procedure & if statement |  |
| >set fileId [open tmp w] anumber >puts $fileId “hello” >close $fileID >type tmp hello > >source Example1.tcl | >proc guess {value} { global sum if {$value < $sum} { puts “too low” } else { if {$value > $sum} { puts “too high” } else { puts “you got it!”} } } > guess 9 too low > |  |

## OpenSees Interpreter

The main abstractions of OpenSees will be explained using the OpenSees interpreter. The interpreter is an extension of the Tcl (page 10) scripting language. The OpenSees interpreter adds commands to Tcl for finite element analysis. Each of these commands is associated (bound) with a C++ procedure that is provided. It is this procedure that is called upon by the interpreter to parse the command. In this document we outline only those commands which have been added to Tcl by OpenSees.

For OpenSees we have added commands to Tcl for finite element analysis:

- Modeling – create nodes, elements, loads and constraints

- Analysis – specify the analysis procedure.

- Output specification – specify what it is you want to monitor during the analysis.

- HELP

http://opensees.berkeley.edu/OpenSees/OpenSeesExamples.pdf (see http://opensees.berkeley.edu/OpenSees/OpenSeesExamples.pdf")

http://opensees.berkeley.edu/cgi-bin/OpenSeesCommands.pl (see http://opensees.berkeley.edu/cgi-bin/OpenSeesCommands.pl")

What is OpenSees?

Chapter 2

# Open System for Earthquake Engineering Simulation

- •	An object-oriented software framework for simulation applications in earthquake engineering using finite element methods. OpenSees is not a code.

- •	A communication mechanism within PEER for exchanging and building upon research accomplishments.

- •	As open-source software, it has the potential for a community code for earthquake engineering.

OpenSees is comprised of a set of modules to perform creation of the finite element model, specification of an analysis procedure, selection of quantities to be monitored during the analysis, and the output of results. In each finite element analysis, an analysis is used to construct 4 main types of objects, as shown in the figure:

- Figure 1: Principal OpenSees Objects

- In This Chapter ModelBuilder Object 14 Domain Object 14 Analysis Object 15 Recorder Object 16

## ModelBuilder Object

The model builder constructs As in any finite element analysis, the analyst's first step is to subdivide the body being studied into elements and nodes, to define loads acting on the elements and nodes, and to define constraints acting on the nodes.

The ModelBuilder is the object in the program responsible for building the following objects in the model and adding them to the domain:

- Node (page 18)

- Mass (page 19)

- Material (page 41, page 25)

- Section (page 47)

- Element (page 60)

- LoadPattern (page 100)

- TimeSeries (page 95)

- Transformation (page 81)

- Block (page 76)

- Constraint (page 109)

## Domain Object

The Domain object is responsible for storing the objects created by the ModelBuilder (page 14) object and for providing the Analysis (page 15) and Recorder (page 16) objects access to these objects.

- Figure 2: Domain Object

## Analysis Object

The Analysis object is responsible for performing the analysis. This may vary from a simple static (page 106) linear analysis to a transient (page 107, page 107) non-linear analysis.  In OpenSees each Analysis object is composed of several component objects, which define how the analysis is performed. The component classes consist of the following:

- ConstraintHandler (page 109)

- DOF_Numberer (page 125)

- AnalysisModel (page 106)

- SolutionAlgorithm (page 119)

- Integrator (page 113)

- SystemOfEqn (page 127)

- Solver (page 127)

- Figure 3: Analysis Object

## Recorder Object

The recorder object monitors user-defined parameters in the model during the analysis. This, for example, could be the displacement history at a node in a transient analysis, or the entire state of the model at each step of the solution procedure. Several Recorder (page 85) objects are created by the analyst to monitor the analysis.

- What does a recorder do?

- Monitors the state of a domain component (node, element, etc.) during an analysis

- Writes this state to a file or to a database at selected intervals during the analysis

- There are also recorders for plotting and monitoring residuals

Once in a file, the information can be easily post-processed.

These objects are used to create the physical model.

Chapter 3

# Model Commands

- In This Chapter Model Command 17 Node Command 18 Mass Command 19 Constraints Commands 20 uniaxialMaterial Command 25 nDMaterial Command 41 section Command 47 element Command 60 block Command 76 region Command 80 Geometric Transformation Command 81 recorder Command 85

## Model Command

This command is used to construct a ModelBuilder object.

Currently there is only one type of ModelBuilder accepted.

For an example of this command, refer to the Model Building Example (page 147)

### Basic Model Builder

This command is used to construct the BasicBuilder object.

model BasicBuilder -ndm $ndm <-ndf $ndf>

| $ndm | dimension of problem (1,2 or 3) |
| --- | --- |
| $ndf | number of degrees of freedom at node (optional) (default value depends on value of ndm: ndm=1 -> ndf=1 ndm=2 -> ndf=3 ndm=3 -> ndf=6) |

These additional commands allow for the construction of Nodes (page 18), Masses (page 19), Materials (page 41, page 25), Sections (page 47), Elements (page 60), LoadPatterns (page 100), TimeSeries (page 95), Transformations (page 81), Blocks (page 76) and Constraints (page 109). These additional commands are described in the subsequent chapters.

EXAMPLE:

model basic -ndm 3 -ndf 6;	# 3 spacial dimensions, 6 DOF's per node

For an example of this command, refer to the Model Building Example (page 147)

### build Command

This command is used to invoke build() (????) on the ModelBuilder (page 14) object.

build

This command has no effect a BasicBuilder (page 17) object, but will on other types of ModelBuilder (page 14) objects.

## Node Command

This command is used to construct a Node object. It assigns coordinates and masses to the Node object.

node $nodeTag (ndm $coords) <-mass (ndf $MassValues)>

| $nodeTag | integer tag identifying node |
| --- | --- |
| $coords | nodal coordinates (ndm (page 17) arguments) |
| $MassValues | nodal mass corresponding to each DOF (ndf (page 17) arguments) (optional) |

The optional -mass string allows analyst the option of associating nodal mass with the node

EXAMPLE:

node  1     0.0     0.0     0.0;	# x,y,z coordinates (0,0,0) of node 1

node  2     0.0     120.     0.0;	# x,y,z coordinates (0,120,0) of node 2

For an example of this command, refer to the Model Building Example (page 147)

## Mass Command

This command is used to set the mass at a node.

mass $nodeTag (ndf $MassValues)

| $nodeTag | integer tag identifying the node associated with the mass |
| --- | --- |
| $MassValues | mass values corresponding to each nodal degrees of freedom (ndf (page 17) values) |

EXAMPLE:

mass 2  2.5 0.0  2.5 0.0 0.0 0.0;	# define mass in x and z coordinates

For an example of this command, refer to the Model Building Example (page 147)

Constraints Commands

Chapter 4

From Cook: " A constraint either prescribes the value of a DOF (as in imposing a support condition) or prescribes a relationship amond DOF. In common terminology, a single-point constraint sets a sing DOF to aknown value (often zero) and a multi-point constraint imposes a relationship between two or more DOF". For example, support conditions on a three-bar truss invoke single-point constraints, while rigid links and rigid elements each invoke a multi-point constraint.

- In This Chapter Single-Point Constraints 20 Multi-Point Constraints 22

### Single-Point Constraints

The following commands construct homogeneous single-point boundary constraints.

#### Fix Command

This command is used to construct homogeneous single-point boundary constraints.

fix $nodeTag (ndf $ConstrValues)

| $nodeTag | integer tag identifying the node to be constrained |  |
| --- | --- | --- |
| $ConstrValues | constraint type (0 or 1). ndf (page 14) values are specified, corresponding to the ndf degrees-of-freedom. The two constraint types are: |  |
|  | 0 | unconstrained |
|  | 1 | constrained |

EXAMPLE:

fix   1   1  1  1  1  1  1;		# node 1: fully fixed

fix 2 0 1 0 0 1 0 -mass 2.5 0.0  2.5 0.0 0.0 0.0;	# node 2: restrain axial elongation and torsion, translational masses in x-z plane only

For an example of this command, refer to the Model Building Example (page 147)

#### fixX Command

This command is used to construct multiple homogeneous single-point boundary constraints for all nodes whose x-coordinate lies within a specified distance from a specified coordinate.

fixX $xCoordinate (ndf $ConstrValues) <-tol $tol>

For example, this command is used when specifying boundary conditions for a series of nodes lying in a plane parallel to the y-z plane in global coordinates.

| $xCoordinate | x-coordinate of nodes to be constrained |  |
| --- | --- | --- |
| $ConstrValues | constraint type (0 or 1). ndf (page 14) values are specified, corresponding to the ndf degrees-of-freedom. The two constraint types are: |  |
|  | 0 | unconstrained |
|  | 1 | constrained |
| $tol | user-defined tolerance (optional, default = 1e-10) |  |

EXAMPLE:

fixX 0.0 1 1 1 1 1 1 -tol 0.1;	# fully restrain all nodes in y-z plane at origin (x=0.0)

#### fixY Command

This command is used to construct multiple homogeneous single-point boundary constraints for all nodes whose y-coordinate lies within a specified distance from a specified coordinate.

fixY $yCoordinate (ndf $ConstrValues) <-tol $tol>

For example, this command is used when specifying boundary conditions for a series of nodes lying in a plane parallel to the x-z plane in global coordinates.

| $yCoordinate | y-coordinate of nodes to be constrained |  |
| --- | --- | --- |
| $ConstrValues | constraint type (0 or 1). ndf values are specified, corresponding to the ndf (page 17) degrees-of-freedom. The two constraint types are: |  |
|  | 0 | unconstrained |
|  | 1 | constrained |
| $tol | user-defined tolerance (optional, default = 1e-10) |  |

EXAMPLE:

fixY 0.0 1 1 1 1 1 1 -tol 0.1;	# fully restrain all nodes in x-z plane at origin (y=0.0)

#### fixZ Command

This command is used to construct multiple homogeneous single-point boundary constraints for all nodes whose z-coordinate lies within a specified distance from a specified coordinate.

fixZ $zCoordinate (ndf $ConstrValues) <-tol $tol>

For example, this command is used when specifying boundary conditions for a series of nodes lying in a plane parallel to the x-y plane in global coordinates.

| $zCoordinate | z-coordinate of nodes to be constrained |  |
| --- | --- | --- |
| $ConstrValues | constraint type (0 or 1). ndf values are specified, corresponding to the ndf (page 17) degrees-of-freedom. The two constraint types are: |  |
|  | 0 | unconstrained |
|  | 1 | constrained |
| $tol | user-defined tolerance (optional, default = 1e-10) |  |

EXAMPLE:

fixZ 0.0 1 1 1 1 1 1 -tol 0.1;	# fully restrain all nodes in x-y plane at origin (z=0.0)

### Multi-Point Constraints

The following commands construct multi-point boundary constraints.

#### equalDOF Command

This command is used to construct a multi-point constraint between nodes.

equalDOF $rNodeTag $cNodeTag $dof1 $dof2 ...

| $rNodeTag | integer tag identifying the retained, or master node (rNode) |
| --- | --- |
| $cNodeTag | integer tag identifying the constrained, or slave node (cNode) |
| $dof1 $dof2 ... | nodal degrees-of-freedom that are constrained at the cNode to be the same as those at the rNode Valid range is from 1 through ndf (page 17), the number of nodal degrees-of-freedom. |

EXAMPLE:

| equalDOF 2 3 1 3 5; | # impose the traslational displacements in x and y directions, and rotation about the y-axis of node 3 to be the same as those of node 2. |
| --- | --- |

#### rigidDiaphragm Command

This command is used to construct a number of Multi-Point Constraint (MP_Constraint) objects. These objects will constraint certain degrees-of-freedom at the listed slave nodes to move as if in a rigid plane with the master node.

rigidDiaphragm $perpDirn $masterNodeTag $slaveNodeTag1 $slaveNodeTag2 ...

| $perpDirn | direction perpendicular to the rigid plane (i.e. direction 3 corresponds to the 1-2 plane) The rigid plane can be the 1-2, 1-3 or 2-3 plane |
| --- | --- |
| $masterNodeTag | integer tag identifying the master node |
| $slaveNodeTag1 $slaveNodeTag2 ... | nodes that are to be constrained to the master node |

NOTE: The constraint object is constructed assuming small rotations.

NOTE: The rigidDiaphragm command works only for problems in three dimensions with six-degrees-of-freedom at the nodes (ndf (page 17) = 6).

EXAMPLE:

rigidDiaphragm 2 2 4 5 6; 	constrain nodes 4,5,6 to move as if in the same x-z plane as node 2.

#### rigidLink Command

This command is used to construct a single MP_Constraint object.

rigidLink $type $masterNodeTag $slaveNodeTag

| $type | string-based argument for rigid-link type: |  |  |
| --- | --- | --- | --- |
|  |  | rod | only the translational degree-of-freedom will be constrained to be exactly the same as those at the master node |
|  |  | beam | both the translational and rotational degrees of freedom are constrained. |
| $masterNodeTag | integer tag identifying the master node |  |  |
| $slaveNodeTag | integer tag identifying the slave node to be constrained to master node |  |  |

NOTE: The constraint object constructed for the beam option assumes small rotations

EXAMPLE:

rigidLink beam 2 3;	# connect node 3 to node 2 via a rigid link-beam.

uniaxialMaterial Command

Chapter 5

This command is used to construct a UniaxialMaterial object which represents uniaxial stress-strain (or force-deformation) relationships.

The valid queries to any uniaxial material when creating an ElementRecorder (page 88) are 'strain,' 'stress,' and 'tangent.'

- In This Chapter Elastic Material 25 Elastic-Perfectly Plastic Material 26 Elastic-Perfectly Plastic Gap Material 27 Parallel Material 28 Series Material 29 Hardening Material 30 Steel01 Material 31 Concrete01 Material 32 Elastic-No Tension Material 33 Hysteretic Material 34 Viscous Material 35 Fedeas Materials 36

### Elastic Material

This command is used to construct an elastic uniaxial material object.

uniaxialMaterial Elastic $matTag $E  <$eta>

| $matTag | unique material object integer tag |
| --- | --- |
| $E | tangent |
| $eta | damping tangent (optional, default=0.0) |

- Figure 4: Elastic Material

### Elastic-Perfectly Plastic Material

This command is used to construct an elastic perfectly-plastic uniaxial material object.

uniaxialMaterial ElasticPP $matTag $E $epsyP <$epsyN $eps0>

| $matTag | unique material object integer tag |
| --- | --- |
| $E | tangent |
| $epsyP | strain or deformation at which material reaches plastic state in tension |
| $epsyN | strain at which material reaches plastic state in compression (optional, default: tension value) |
| $eps0 | initial strain (optional, default: zero) |

- Figure 5: Elastic-Perfectly Plastic Material

### Elastic-Perfectly Plastic Gap Material

This command is used to construct an elastic perfectly-plastic gap uniaxial material object.

uniaxialMaterial ElasticPPGap $matTag $E $Fy $gap

| $matTag | unique material object integer tag |
| --- | --- |
| $E | tangent stiffness |
| $Fy | stress or force at which material reaches plastic state |
| $gap | initial gap (strain or deformation) |

NOTE: To create a compression-only gap element, NEGATIVE values need to be specified for $Fy and $gap.

- Figure 6: Elastic-Perfectly Plastic Gap Material

### Parallel Material

This command is used to construct a parallel material object made up of an arbitrary number of previously-constructed UniaxialMaterial (page 25) objects.

uniaxialMaterial Parallel $matTag $tag1 $tag2 ...

| $matTag | unique material object integer tag |
| --- | --- |
| $tag1 $tag2 ... | identification of materials making up the material model |

The parallel material is represented graphically:

- Figure 7: Parallel Material

In a parallel model, strains are equal and stresses and stiffnesses are additive:

### Series Material

This command is used to construct a series material object made up of an arbitrary number of previously-constructed UniaxialMaterial (page 25) objects.

uniaxialMaterial Series $matTag $tag1 $tag2 ...

| $matTag | unique material object integer tag |
| --- | --- |
| $tag1 $tag2 ... | identification of materials making up the material model |

The series material is represented graphically:

- Figure 8: Series Material

In a series model, stresses are equal and strains and flexibilities are additive:

- Figure 9: Series Material Relationship

### Hardening Material

This command is used to construct a uniaxial material object with combined linear kinematic and isotropic hardening. The model includes optional visco-plasticity using a Perzyna formulation (REF???)

uniaxialMaterial Hardening $matTag $E $sigmaY $H_iso $H_kin <$eta>

| $matTag | unique material object integer tag |
| --- | --- |
| $E | tangent stiffness |
| $sigmaY | yield stress or force |
| $H_iso | isotropic hardening Modulus |
| $H_kin | kinematic hardening Modulus |
| $eta | visco-plastic coefficient (optional, default=0.0) |

- Figure 10: Hardening Material

### Steel01 Material

This command is used to construct a uniaxial bilinear steel material object with kinematic hardening and optional isotropic hardening described by a non-linear evolution equation (REF: Fedeas).

uniaxialMaterial Steel01 $matTag $Fy $E0 $b <$a1 $a2 $a3 $a4>

| $matTag | unique material object integer tag |
| --- | --- |
| $Fy | yield strength |
| $E0 | initial elastic tangent |
| $b | hardening ratio |
| $a1, $a2, $a3, $a4 | isotropic hardening parameters: (optional, default: no isotropic hardening) |

- Figure 11: Steel01 Material

### Concrete01 Material

This command is used to construct a uniaxial Kent-Scott-Park concrete material object with degraded linear unloading/reloading stiffness according to the work of Karsan-Jirsa and no tensile strength. (REF: Fedeas).

uniaxialMaterial Concrete01 $matTag $fpc $epsc0 $fpcu $epsU

| $matTag | unique material object integer tag |
| --- | --- |
| $fpc | compressive strength* |
| $epsc0 | strain at compressive strength* |
| $fpcu | crushing strength* |
| $epsU | strain at crushing strength* |

*NOTE: Compressive concrete parameters should be input as negative values.

The initial slope for this model is (2*$fpc/$epsc0)

- Figure 12: Concrete01 Material

### Elastic-No Tension Material

This command is used to construct a uniaxial elastic-no tension material object.

uniaxialMaterial ENT $matTag $E

| $matTag | unique material object integer tag |
| --- | --- |
| $E | elastic model in compression |

In tension, there is zero stress.

- Figure 13: Elastic-No Tension Material

### Hysteretic Material

This command is used to construct a uniaxial bilinear hysteretic material object with pinching of force and deformation, damage due to ductility and energy, and degraded unloading stiffness based on ductility.

uniaxialMaterial Hysteretic $matTag $s1p $e1p $s2p $e2p <$s3p $e3p> $s1n $e1n $s2n $e2n <$s3n $e3n> $pinchX $pinchY $damage1 $damage2 <$beta>

| $matTag |  | unique material object integer tag |
| --- | --- | --- |
| $s1p | $e1p | stress and strain (or force & deformation) at first point of the envelope in the positive direction |
| $s2p | $e2p | stress and strain (or force & deformation) at second point of the envelope in the positive direction |
| $s3p | $e3p | stress and strain (or force & deformation) at third point of the envelope in the positive direction (optional) |
| $s1n | $e1n | stress and strain (or force & deformation) at first point of the envelope in the negative direction* |
| $s2n | $e2n | stress and strain (or force & deformation) at second point of the envelope in the negative direction* |
| $s3n | $e3n | stress and strain (or force & deformation) at third point of the envelope in the negative direction (optional)* |
| $pinchX |  | pinching factor for strain (or deformation) during reloading |
| $pinchY |  | pinching factor for stress (or force) during reloading |
| $damage1 |  | damage due to ductility: D1(mu-1) |
| $damage2 |  | damage due to energy: D2(Eii/Eult) |
| $beta |  | power used to determine the degraded unloading stiffness based on ductility, mu-beta (optional, default=0.0) |

*NOTE: negative backbone points should be entered as negative numeric values

- Figure 14: Hysteretic Material

### Viscous Material

This command is used to construct a uniaxial material object with a non-linear elastic stress-strain-rate relation given by:

stress =C(strain-rate)alpha.

uniaxialMaterial Viscous $matTag $C $alpha

| $matTag | unique material object integer tag |
| --- | --- |
| $C | tangent |
| $alpha | damping tangent |

### Fedeas Materials

This section lists the uniaxial material objects available from the Fedeas ML1D library developed by F.C. Filippou. For more information see the Fedeas materials web page:

http://www.ce.berkeley.edu/~filippou/Research/Fedeas/material.htm (see http://www.ce.berkeley.edu/~filippou/Research/Fedeas/material.htm")

Further information on the Concrete01 (page 32) and Steel01 (page 31) materials described earlier in this document can also be found at this web page. Currently, each of the following Fedeas materials are available only on the Win32 version of OpenSees

#### Concrete02 Material

This command is used to construct a uniaxial concrete material object with tensile strength and linear tension softening.

uniaxialMaterial Concrete02 $matTag $fpc $epsc0 $fpcu $epscu $ratio $ft $Ets

| $matTag | unique material object integer tag |
| --- | --- |
| $fpc | compressive strength* |
| $epsc0 | strain at compressive strength* |
| $fpcu | crushing strength* |
| $epsU | strain at crushing strength* |
| $ratio | ratio between unloading slope at $epscu and initial slope |
| $ft | tensile strength |
| $Ets | slope of the linear tension softening branch |

*NOTE: Compressive concrete parameters should be input as negative values.

The initial slope for this model is (2*$fpc/$epsc0)

- Figure 15: Concrete02 Material

#### Concrete03 Material

This command is used to construct a uniaxial concrete material object with tensile strength and nonlinear tension softening.

uniaxialMaterial Concrete03 $matTag $fpc $epsc0 $fpcu $epscu $ratio $ft $epst0 $ft0 $beta $epstu

| $matTag | unique material object integer tag |
| --- | --- |
| $fpc | compressive strength* |
| $epsc0 | strain at compressive strength* |
| $fpcu | crushing strength* |
| $epsU | strain at crushing strength* |
| $ratio | ratio between unloading slope at $epscu and initial slope (=2*$fpc/$epsc0) |
| $ft | tensile strength |
| $epst0 | tensile strain at the transition from nonlinear to linear softening |
| $ft0 | tensile stress at the transition from nonlinear to linear softening |
| $beta | exponent of the tension softening curve |
| $epstu | ultimate tensile strain |

*NOTE: Compressive concrete parameters should be input as negative values.

- Figure 16: Concrete03 Material

#### Steel02 Material

This command is used to construct a uniaxial Menegotto-Pinto steel material object with isotropic strain hardening.

uniaxialMaterial Steel02 $matTag $Fy $E $b $R0 $cR1 $cR2 $a1 $a2 $a3 $a4

| $matTag |  |  |  | unique material object integer tag |
| --- | --- | --- | --- | --- |
| $Fy |  |  |  | yield strength |
| $E |  |  |  | initial elastic tangent |
| $b |  |  |  | strain hardening ratio |
| $R0 | $cR1 | $cR2 |  | control the transition from elastic to plastic branches. Recommended values: $R0=18.5, $cR1=0.925, $cR2=0.15 |
| $a1 | $a2 | $a3 | $a4 | isotropic hardening parameters: no isotropic hardening: a1=a3=0; a2, a4 =nonzero |

#### Bond01 Material

This command is used to construct an Eligehousen bond material object without damage.(REF: Fedeas).

uniaxialMaterial Bond01 $matTag $u1p $q1p $u2p $u3p $q3p $u1n $q1n $u2n $u3n $q3n $s0 $bb

Tensile bond-slip backbone parameter

| $matTag |  | unique material object integer tag |
| --- | --- | --- |
| $u1p | $q1p | slip and bond at first detachment -- tensile bond-slip backbone |
| $u2p |  | slip at start of degradation -- tensile bond-slip backbone |
| $u3p | $q3p | slip and bond at ultimate -- tensile bond-slip backbone |
| $u1n | $q1n | slip and bond at first detachment -- compressive bond-slip backbone* |
| $u2n |  | slip at start of degradation -- compressive bond-slip backbone* |
| $u3n | $q3n | slip and bond at ultimate -- compressive bond-slip backbone* |
| $s0 |  | unloading stiffness |
| $bb |  | exponent for the first branch of the backbone (prior to first detachment). i.e. q=ubb |

*NOTE: Compressive concrete parameters should be input as negative values.

#### Bond02 Material

This command is used to construct an Eligehousen bond material object with damage.

uniaxiaMaterial Bond02 $matTag $u1p $q1p $u2p $u3p $q3p $u1n $q1n $u2n $u3n $q3n $s0 $bb $alp $aln

| $matTag |  | unique material object integer tag |
| --- | --- | --- |
| $u1p | $q1p | slip and bond at first detachment -- tensile bond-slip backbone |
| $u2p |  | slip at start of degradation -- tensile bond-slip backbone |
| $u3p | $q3p | slip and bond at ultimate -- tensile bond-slip backbone |
| $u1n | $q1n | slip and bond at first detachment -- compressive bond-slip backbone* |
| $u2n |  | slip at start of degradation -- compressive bond-slip backbone* |
| $u3n | $q3n | slip and bond at ultimate -- compressive bond-slip backbone* |
| $s0 |  | unloading stiffness |
| $bb |  | exponent for the first branch of the backbone (prior to first detachment). i.e. q=u$bb |
| $alp |  | damage factor for positive quadrant |
| $aln |  | damage factor for negative quadrant |

*NOTE: Compressive concrete parameters should be input as negative values.

nDMaterial Command

Chapter 6

This command is used to construct an NDMaterial object which represents stress-strain relationships at the integration points of continuum and force-deformation elements.

The valid queries to any ND material when creating an ElementRecorder (page 88) are 'strain,' 'stress,' and 'tangent.'

- In This Chapter Elastic Isotropic Material 41 J2 Plasticity Material 41 Plane Stress Material 42 Plate Fiber Material 42 Template Elasto-Plastic Material 43

### Elastic Isotropic Material

This command is used to construct an ElasticIsotropic material object.

nDMaterial ElasticIsotropic $matTag $E $v

| $matTag | unique material object integer tag |
| --- | --- |
| $E | elastic Modulus |
| $v | Poisson's ratio |

The material formulations for the ElasticIsotropic object are "ThreeDimensional," "PlaneStrain," "Plane Stress," "AxiSymmetric," and "PlateFiber." These are the valid strings that can be passed to the continuum elements (page 71, page 69, page 70, page 68, page 69, page 70) for the type parameter.

### J2 Plasticity Material

This command is used to construct a J2Plasticity material object.

nDmaterial J2Plasticity $matTag $K $G $sig0 $sigInf $delta $H

| $matTag | unique material object integer tag |
| --- | --- |
| $K | bulk Modulus |
| $G | shear Modulus |
| $sig0 | initial yield stress |
| $sigInf | final saturation yield stress |
| $delta | exponential hardening parameter |
| $H | linear hardening parameter |

### Plane Stress Material

This command is used to construct a plane-stress material wrapper which converts any three-dimensional material into a plane stress material via static condensation.

nDMaterial PlaneStress $matTag $threeDtag

| $matTag | unique material object integer tag |
| --- | --- |
| $threeDTag | material tag for a previously-defined three-dimensional material |

### Plate Fiber Material

This command is used to construct a plate-fiber material wrapper which converts any three-dimensional material into a plate fiber material (by static condensation) appropriate for shell analysis.

nDMaterial PlateFiber $matTag $threeDTag

| $matTag | unique material object integer tag |
| --- | --- |
| $threeDTag | material tag for a previously-defined three-dimensional material |

### Template Elasto-Plastic Material

This command is used to construct the template elasto-plastic mateial object.

nDMaterial Template3Dep $matTag -YS $ys -PS $ps -EPS $eps -ELS1 $el <-ELT1 $et>

| $matTag | unique material object tag |
| --- | --- |
| $ys | yield surface variable, previously defined in Yield Surface (page 43) object |
| $ps | potential surface variable, previously defined in Potential Surface (page 44) object |
| $eps | elasto-plastic state variable, previously defined in EPState (page 46) object |
| $el | scalar evolution law variable, previously defined in Evolution Law (page 45) object |
| $et | tensorial envolution law variable, previously defined in Evolution Law (page 45) object |

#### Yield Surface

This command sets the yield surface variable ys to be the specified type. Currently these  include: Drucker-Prager yield surface, von Mises yield surface and Cam-Clay yield surface.

set ys "-YieldSurfaceType <parameter list>"

valid strings for YieldSurfaceType are DP, VM and CC.

- For Drucker-Prager yield surface

set ys "-DP"

- For von Mises yield surface

set ys "-VM"

- For Cam-Clay yield surface

set "-CC $M"

| $M | slope of the critical state line in p-q space |
| --- | --- |

#### Potential Surface

This command is used to set the potential surface variable $ps to the specific surface (or directly to the flow directions). Currently included are: Drucker-Prager potential surface, von Mises potential surface, Cam-Clay potential surface and Manzari Dafalias flow directions.

set ps "-PotentialSurfaceType <parameter list>"

Valid strings for PotentialSurfaceType are DP, VM and CC.

- For the Drucker-Prager potential surface

set ps "-DP"

- For the von Mises potential surface

set  ps "-VM"

- For the Cam-Clay potential surface

set ps "-CC $M"

| $M | slope of the critical state line in p-q space |
| --- | --- |

#### Evolution Law

This command is used to set the evolution law variable el to the specified type. There are two types of evolutions laws implemented: scalar evolution and tensorial evolution. For scalar evolution law, there are linear scalar evolution law and  nonlinear scalar evolution law. For tensorial evolution law, there are  linear tensorial evolution law and nonlinear tensorial evolution law.

set el "-EvolutionLawType <parameter list>"

Valid strings for EvolutionLawType are Leq, NLp, LEij, NLEij

- For linear scalar evolution law

set el "-Leq $a"

- For nonlinear scalar evolution law

set el "-NLp $e_o $lambda $k

- For linear tensorial evolution law

set et "-LEij $a1"

- For nonlinear tensorial evolution law

set et "-NLEij $h_a $C_r"

| $a | linear hardening coefficient |
| --- | --- |
| $e_o | initial void ratio |
| $lambda | nonlinear evolution law constant (Cam Clay type) |
| $k | nonlinear evolution law constant (Cam Clay type) |
| $a1 | linear tensorial evolution law constant |
| $h_a | nonlinear tensorial evolution law constant (Armstrong, Frederick type) |
| $C_r | nonlinear tensorial evolution law constant (Armstrong, Frederick type) |

#### EPState

This command is used to set the Elasto-Plastic State which include two states.

- To set the initial stress tensor to variable sts:

set sts "$sigma_xx $sigma_xy $sigma_xz $sigma_yx $sigma_yy $sigma_yz $sigma_zx $sigma_zy $sigma_zz"

- To assign to the Elasto-Plastic State variable eps the specified state parameters

set eps "$Eo $E $v $p -NOD $nt -NOS $ns $scalar1 $scalar2 ... -stressp $sts"

| $sigma_xx $sigma_xy $sigma_xz $sigma_yx $sigma_yy $sigma_yz $sigma_zx $sigma_zy $sigma_zz | Initial stress tensor components (Default = 0.0) |
| --- | --- |
| $Eo | Young's Modulus at atmospheric pressure |
| $E | current Young's Modulus E |
| $v | Poisson's ratio |
| $p | mass density (mass/volume) |
| $nt | number of tensorial internal variables |
| $ns | number of scalar internal variables |
| $scalar1 $scalar2 ... | corresponding intial values of scalar internal variables |
| $sts | intial stresses |

section Command

Chapter 7

This command is used to construct a SectionForceDeformation object, hereto referred to as Section, which represents force-deformation (or resultant stress-strain) relationships at beam-column and plate sample points.

- What is a section?

- A section defines the stress resultant force-deformation response at a cross section of a beam-column or plate element

- Types of sections:

•	Elastic – defined by material and geometric constants

•	Resultant – general nonlinear description of force-deformation response, e.g. moment-curvature

•	Fiber – section is discretized into smaller regions for which the material stress-strain response is integrated to give resultant behavior, e.g. reinforced concrete

The valid queries to any section when creating an ElementRecorder (page 88) are 'force' and 'deformation.'

- Figure 17: Section Representation

- In This Chapter Elastic Section 48 Uniaxial Section 49 Fiber Section 50 Section Aggregator 56 Elastic Membrane Plate Section 58 Plate Fiber Section 59 Bidirectional Section 59

### Elastic Section

This command is used to construct an ElasticSection object.

section Elastic $secTag $E $A $Iz <$Iy $G $J>

| $secTag | unique section object tag |
| --- | --- |
| $E | Young's Modulus |
| $A | cross-sectional area of section |
| $Iz | second moment of area about the local z-axis |
| $Iy | second moment of area about the local y-axis (optional, used for 3D analysis) |
| $G | Shear Modulus (optional, used for 3D analysis) |
| $J | torsional moment of inertia of section (optional, used for 3D analysis) |

This command is useful for patch tests of the nonlinear beam-column elements (page 63). It also allows nonlinear beam-column elements to be used for elastic analysis.

EXAMPLE:

section Elastic 1 29000 100 100000 80000 20000 100000;	# create elastic section with IDtag 1

### Uniaxial Section

This command is used to construct a UniaxialSection object which uses a previously-defined UniaxialMaterial (page 25) object to represent a single section force-deformation response quantity. (Formerly known as Generic1d section, which is still accepted by OpenSees)

section Uniaxial $secTag $matTag $string

| $secTag |  | unique section object tag |  |
| --- | --- | --- | --- |
| $matTag |  | previously-defined UniaxialMaterial (page 25) object |  |
| $string |  | the force-deformation quantity to be modeled by this section object. One of the following strings is used: |  |
|  | P |  | Axial force-deformation |
|  | Mz |  | Moment-curvature about section local z-axis |
|  | Vy |  | Shear force-deformation along section local y-axis |
|  | My |  | Moment-curvature about section local y-axis |
|  | Vz |  | Shear force-deformation along section local z-axis |
|  | T |  | Torsion Force-Deformation |

EXAMPLE:

section Uniaxial 1 1 Mz;	# create sectionID-tag 1 from UniaxialMaterialID-tag 1 for the moment-curvature about section local z-axis.

### Fiber Section

The FiberSection object is composed of Fiber objects.

A fiber section has a general geometric configuration formed by subregions of simpler, regular shapes (e.g. quadrilateral, circular and triangular regions) called patches. In addition, layers of reinforcement bars can be specified. The subcommands patch (page 52) and layer (page 55, page 54) are used to define the discretization of the section into fibers. Individual fibers, however, can also be defined using the fiber (page 51) command. During generation, the Fiber objects are associated with uniaxialMaterial (page 25) objects, which enforce Bernoulli beam assumptions.

The geometric parameters are defined with respect to a planar local coordinate system (y,z). See figures.

section Fiber $secTag {

fiber <fiber arguments>

patch <patch arguments>

layer <layer arguments>

}

An example fiber section is shown in the Figure.

- Figure 18: Fiber Section

#### Fiber Command

This command is used to construct a UniaxialFiber object and add it to the section.

fiber $yLoc $zLoc $A $matTag

| $yLoc | y coordinate of the fiber in the section (local coordinate system) |
| --- | --- |
| $zLoc | z coordinate of the fiber in the section (local coordinate system) |
| $A | area of fiber |
| $matTag | material tag of the pre-defined UniaxialMaterial (page 25) object used to represent the stress-strain for the area of the fiber |

NOTE: in 2D (page 17) bending is about the local z-axis

EXAMPLE:

fiber 0.0 0.0 1.0 1;	# create a single fiber of area 1.0 at the origin (0,0) of the section, using materialIDtag 1

- Figure 19: Fiber Command

#### Quadrilateral Patch Command

This command is used to construct a Patch object with a quadrilateral shape. The geometry of the patch is defined by four vertices: I J K L, as illustrated in the Figure. The coordinates of each of the four vertices is specified in sequence -- counter-clockwise.

patch quad $matTag $numSubdivIJ $numSubdivJK $yI $zI $yJ $zJ $yK $zK $yL $zL

| $matTag |  | material integer tag of the previously-defined UniaxialMaterial (page 25) object used to represent the stress-strain for the area of the fiber |
| --- | --- | --- |
| $numSubdivIJ |  | number of subdivisions (fibers) in the IJ direction. |
| $numSubdivJK |  | number of subdivisions (fibers) in the JK direction. |
| $yI | $zI | y & z-coordinates of vertex I (local coordinate system) |
| $yJ | $zJ | y & z-coordinates of vertex J (local coordinate system) |
| $yK | $zK | y & z-coordinates of vertex K (local coordinate system) |
| $yL | $zL | y & z-coordinates of vertex L (local coordinate system) |

NOTE: in 2D (page 17) bending is about the local z-axis

EXAMPLE:

patch quad $coreMatTag 8 8 -$b -$h $b -$h $b $h -$b $h;   # define core patch with 8 subdivisions within a rectange of width 2b and depth 2h

- Figure 20: Quadrilateral Patch

#### Circular Patch Command

This command is used to construct a Patch object with a circular shape.

patch circ $matTag $numSubdivCirc $numSubdivRad $yCenter $zCenter $intRad $extRad <$startAng $endAng>

| $matTag |  | material integer tag of the previously-defined UniaxialMaterial (page 25) object used to represent the stress-strain for the area of the fiber |
| --- | --- | --- |
| $numSubdivCirc |  | number of subdivisions (fibers) in the circumferential direction. |
| $numSubdivRad |  | number of subdivisions (fibers) in the radial direction. |
| $yCenter | $zCenter | y & z-coordinates of the center of the circle |
| $intRad |  | internal radius |
| $extRad |  | external radius |
| $startAng |  | starting angle (optional. default=0.0) |
| $endAng |  | ending angle (optional. default=360.0) |

NOTE: in 2D (page 17) bending is about the local z-axis

EXAMPLE:

patch circ $coreMatTag 8 8 0.0 0.0 0.0 $h;   # define core patch with 8 subdivisions within a whole circle of diameter 2h

- Figure 21: Circular Patch

#### Straight Layer Command

This command is used to construct a straight layer of reinforcing bars.

layer straight $matTag $numBars $areaBar $yStart $zStart $yEnd $zEnd

| $matTag |  | material integer tag of the previously-defined UniaxialMaterial (page 25) object used to represent the stress-strain for the area of the fiber |
| --- | --- | --- |
| $numBars |  | number of reinforcing bars along layer |
| $areaBar |  | area of individual reinforcing bar |
| $yStart | $zStart | y and z-coordinates of starting point of reinforcing layer (local coordinate system) |
| $yEnd | $zEnd | y and z-coordinates of ending point of reinforcing layer (local coordinate system) |

NOTE: in 2D (page 17) bending is about the local z-axis

EXAMPLE:

layer straight $steelMatTag 10 0.11 -b -h b -h;   # define steel layer of 10 bars with area 0.11 at bottom of section of width 2b by 2h

- Figure 22: Straight Layer

#### Circular Layer Command

This command is used to construct a circular layer of reinforcing bars.

layer circ $matTag $numBar $areaBar $yCenter $zCenter $radius <$startAng $endAng>

| $matTag |  | material integer tag of the previously-defined UniaxialMaterial (page 25) object used to represent the stress-strain for the area of the fiber |
| --- | --- | --- |
| $numBar |  | number of reinforcing bars along layer |
| $areaBar |  | area of individual reinforcing bar |
| $yCenter | $zCenter | y and z-coordinates of center of reinforcing layer (local coordinate system) |
| $radius |  | radius of reinforcing layer |
| $startAng | $endAng | starting and ending angle of reinforcing layer, respectively. (Optional, Default: a full circle is assumed 0-360) |

NOTE: in 2D (page 17) bending is about the local z-axis

EXAMPLE:

layer circ $steelMatTag 10 0.11 0.0 0.0 $h 0 360;   # define circular steel layer of 10 bars with area 0.11 uniformly distributed along circumference of circle of diameter 2h

- Figure 23: Circular Reinforcing Layer

### Section Aggregator

This command is used to construct a SectionAggregator object which groups previously-defined UniaxialMaterial (page 25) objects into a single section force-deformation model.

section Aggregator $secTag $matTag1 $string1 $matTag2 $string2 ....... <-section $sectionTag>

| $secTag |  | unique section object integer tag |  |
| --- | --- | --- | --- |
| $matTag1, $matTag2 ... |  | previously-defined UniaxialMaterial (page 25) objects |  |
| $string1, $string2 .... |  | the force-deformation quantities corresponding to each section object. One of the following strings is used: |  |
|  | P |  | Axial force-deformation |
|  | Mz |  | Moment-curvature about section local z-axis |
|  | Vy |  | Shear force-deformation along section local y-axis |
|  | My |  | Moment-curvature about section local y-axis |
|  | Vz |  | Shear force-deformation along section local z-axis |
|  | T |  | Torsion Force-Deformation |
| <-section $sectionTag> |  | specifies a previously-defined Section (page 47) object (identified by the argument $sectionTag) to which these UniaxialMaterial (page 25) objects may be added to recursively define a new Section (page 47) object |  |

NOTE: The UniaxialMaterial (page 25) objects aggregated in this Section (page 47) object are uncoupled from each other as well as from the Section (page 47) object represented by $sectionTag, if present.

There are two main tasks that can be performed using the Section Aggregator:

- 1.  Group previously defined uniaxial materials to describe stress resultant section behavior

EXAMPLE:

- Figure 24: Section Aggregator 1

section Aggregator 1 2 Vy 5 Mz; #create new section with IDtag 1, taking the existing material tag 2 to represent the shear and the existing material tag 5 to represent the moment.

- 2.  Add to an existing section

- Figure 25: Section Aggregator 2

EXAMPLE:

section Aggregator 2 2 Vy -section 4; # create new section with IDtag 2, taking the existing material tag 2 to represent the shear and adding it to the existing section tag 4, which may be a fiber section where the interaction betweeen axial force and flexure is already considered.

### Elastic Membrane Plate Section

This command is used to construct an ElasticMembranePlateSection object, which is an isotropic section appropriate for plate and shell analysis.

section ElasticMembranePlateSection $secTag $E $nu $h $rho

| $secTag | unique section object tag |
| --- | --- |
| $E | Elastic Modulus |
| $nu | Poisson's Ratio |
| $h | thickness of the plate section |
| $rho | mass density of the material (per unit volume) |

### Plate Fiber Section

The plate fiber section takes any plate fiber material (page 42) and, by thickness integration, creates a plate section appropriate for shell analysis.

section PlateFiber $secTag $fiberTag $h

| $secTag | unique section object tag for section being constructed |
| --- | --- |
| $fiberTag | material tag for a previously-defined plate fiber material (page 42) |
| $h | thickness of the plate section |

### Bidirectional Section

This command is used to construct a Bidirectional section object which is the two-dimensional generalization of a one-dimensional elasto-plastic model with linear hardening.

section Bidirectional $matTag $E $sigY $H_iso $H_kin

| $matTag | unique section object integer tag |
| --- | --- |
| $E | Elastic Modulus |
| $sigY | yield stress |
| $H_iso | isotropic hardening Modulus |
| $H_kin | kinematic hardening Modulus |

then why do we have kinematic and isotripic hardening parameters??????***********

element Command

Chapter 8

This command is used to construct an Element object.

- In This Chapter Truss Element 60 Corotational Truss Element 61 Elastic Beam Column Element 62 NonLinear Beam-Column Elements 63 Zero-Length Elements 66 Quadrilateral Elements 68 Brick Elements 70

### Truss Element

This command is used to construct a truss element object. There are two ways to construct a truss element object:

One way is to specify an area and a UniaxialMaterial (page 25) identifier:

element truss $eleTag $iNode $jNode $A $matTag

the other is to specify a Section (page 47) identifier:

element truss $eleTag $iNode $jNode $secTag

| $eleTag | unique element object tag |
| --- | --- |
| $iNode $jNode | end nodes |
| $A | cross-sectional area of element |
| $matTag | tag associated with previously-defined UniaxialMaterial (page 25) |
| $secTag | tag associated with previously-defined Section (page 47) |

When constructed with a UniaxialMaterial (page 25) object, the truss element considers strain-rate effects, and is thus suitable for use as a damping element.

The valid queries to a truss element when creating an ElementRecorder (page 88) object are 'axialForce,' 'stiff,' material matArg1 matArg2...,' 'section sectArg1 sectArg2...' There will be more queries after the interface for the methods involved have been developed further.

### Corotational Truss Element

This command is used to construct a Corotational Truss (CorotTruss) element object.  A corotational formulation adopts a set of corotational axes which rotate with the element, thus taking into account an exact geometric transformation between local and global frames of reference.

There are two ways to construct a Corotational Truss element object:

One way is to specify an area and a UniaxialMaterial (page 25) identifier:

element corotTruss $eleTag $iNode $jNode $A $matTag

the other is to specify a Section (page 47) identifier:

element corotTruss $eleTag $iNode $jNode $secTag

| $eleTag | unique element object tag |
| --- | --- |
| $iNode $jNode | end nodes |
| $A | cross-sectional area of element |
| $matTag | tag associated with previously-defined UniaxialMaterial (page 25) object |
| $secTag | tag associated with previously-defined Section (page 47) object |

NOTE: When constructed with a UniaxialMaterial (page 25) object, the truss element considers strain-rate effects, and is thus suitable for use as a damping element.

The valid queries to a corotational truss element when creating an ElementRecorder (page 88) object are 'axialForce,' 'stiff,' 'material $matNum matArg1 matArg2...,' 'section $secNum sectArg1 sectArg2...'

### Elastic Beam Column Element

This command is used to construct an elasticBeamColumn element object. The arguments for the construction of an elastic beam-column element depend on the dimension of the problem, ndm (page 14):

For a two-dimensional problem:

element elasticBeamColumn $eleTag $iNode $jNode $A $E $Iz $transfTag

For a three-dimensional problem:

element elasticBeamColumn $eleTag $iNode $jNode $A $E $G $J $Iy $Iz $transfTag

| $eleTag |  | unique element object tag |
| --- | --- | --- |
| $iNode | $jNode | end nodes |
| $A |  | cross-sectional area of element |
| $E |  | Young's Modulus |
| $G |  | Shear Modulus |
| $J |  | torsional moment of inertia of cross section |
| $Iz |  | second moment of area about the local z-axis |
| $Iy |  | second moment of area about the local y-axis |
| $transfTag |  | identifier for previously-defined coordinate-transformation (page 81) (CrdTransf) object |

The valid queries to an elastic beam-column element when creating an ElementRecorder (page 88) object are 'stiffness' and 'force.'

### NonLinear Beam-Column Elements

There are basically two types of Nonlinear Beam-Column Elements

- Force based elements

- Distributed plasticity (nonlinearBeamColumn (page 63))

- Concentrated plasticity with elastic interior (beamWithHinges (page 64))

- Displacement based element

- Distributed plasticity with linear curvature distribution (dispBeamColumn (page 65))

************NEED FIGURE BY MHS

#### Nonlinear Beam Column Element

This command is used to construct a nonlinearBeamColumn element object, which is based on the non-iterative (or iterative) force formulation, and considers the spread of plasticity along the element.

element nonlinearBeamColumn $eleTag $iNode $jNode $numIntgrPts $secTag $transfTag <-mass $massDens> <-iter $maxIters $tol>

| $eleTag |  | unique element object tag |
| --- | --- | --- |
| $iNode | $jNode | end nodes |
| $numIntgrPts |  | number of integration points along the element. |
| $secTag |  | identifier for previously-defined section (page 47) object |
| $transfTag |  | identifier for previously-defined coordinate-transformation (page 81) (CrdTransf) object |
| $massDens |  | element mass density (per unit length), from which a lumped-mass matrix is formed (optional, default=0.0) |
| $maxIters |  | maximum number of iterations to undertake to satisfy element compatibility (optional, default=1) |
| $tol |  | tolerance for satisfaction of element compatibility (optional, default=10-16) |

The integration along the element is based on Gauss-Lobatto quadrature rule (two integration points at the element ends).

The element is prismatic, i.e. the beam is represented by the section (page 47) model identified by $secTag at each integration point.

The -iter switch enables the iterative form of the flexibility formulation. Note that the iterative form can improve the rate of global convergence at the expense of more local element computation.

The valid queries to a nonlinear beam-column element when creating an ElementRecorder (page 88) object are 'force,' 'stiffness,' and 'section $secNum secArg1 secArg2...' Where $secNum refers to the integration point whose data is to be output.

#### Beam With Hinges Element

This command is used to construct a beamWithHinges element object, which is based on the non-iterative (or iterative) flexibility formulation, and considers plasticity to be concentrated over specified hinge lengths at the element ends.

The arguments for the construction of the element depend on the dimension of the problem, ndm (page 14).

For a two-dimensional problem:

element beamWithHinges $eleTag $iNode $jNode $secTagI $ratioI $secTagJ $ratioJ $E $A $Iz $transfTag <-mass $massDens> <-iter $maxIters $tol>

For a three-dimensional problem:

element beamWithHinges $eleTag $iNode $jNode $secTagI $ratioI $secTagJ $ratioJ $E $A $Iz $Iy $G $J $transfTag <-mass $massDens> <-iter $maxIters $tol>

| $eleTag |  | unique element object tag |
| --- | --- | --- |
| $iNode | $jNode | end nodes |
| $secTagI |  | identifier for previously-defined section (page 47) object corresponding to node I |
| $ratioI |  | ratio of hinge length to total element length at node I |
| $secTagJ |  | identifier for previously-defined section (page 47) object corresponding to node J |
| $ratioJ |  | ratio of hinge length to total element length at node J |
| $E |  | Young's Modulus |
| $A |  | area of element cross-section |
| $Iz |  | section moment of inertia about the section local z-axis |
| $Iy |  | section moment of inertia about the section local y-axis |
| $G |  | Shear Modulus |
| $J |  | torsional moment of inertia of cross section |
| $transfTag |  | identifier for previously-defined coordinate-transformation (page 81) (CrdTransf) object |
| $massDens |  | element mass density (per unit length), from which a lumped-mass matrix is formed (optional, default=0.0) |
| $maxIters |  | maximum number of iterations to undertake to satisfy element compatibility (optional, default=1) |
| $tol |  | tolerance for satisfaction of element compatibility (optional, default=10-16) |

The -iter switch enables the iterative form of the flexibility formulation. Note that the iterative form can improve the rate of global convergence at the expense of more local element computation.

NOTE: The elastic properties are integrated only over the beam interior, which is considered to be linear-elastic. Forces and deformations of the inelastic regions are sampled at the hinge midpoints, using mid-point integration.

The valid queries to a beamWithHinges element when creating an ElementRecorder (page 88) object are 'force,' 'stiffness,' 'rotation' (hinge rotation), or 'section $secNum secArg1 secArg2...' Where $secNum refers to the integration point whose data is to be output.

#### Displacement-Based Beam-Column Element

This command is used to construct a dispBeamColumn element object, which is a distributed-plasticity, displacement-based beam-column element.

element dispBeamColumn $eleTag $iNode $jNode $numIntgrPts $secTag $transfTag <-mass $massDens>

| $eleTag |  | unique element object tag |
| --- | --- | --- |
| $iNode | $jNode | end nodes |
| $numIntgrPts |  | number of integration points along the element. |
| $secTag |  | identifier for previously-defined section (page 47) object |
| $transfTag |  | identifier for previously-defined coordinate-transformation (page 81) (CrdTransf) object |
| $massDens |  | element mass density (per unit length), from which a lumped-mass matrix is formed (optional, default=0.0) |

The integration along the element is based on the Gauss-Legendre quadrature rule (REF???).

The element is prismatic, i.e. the beam is represented by the section model identified by $secTag at each integration point.

The valid queries to a displacement-based beam-column element when creating an ElementRecorder (page 88) object are 'force,' 'stiffness,' and 'section $secNum secArg1 secArg2...' Where $secNum refers to the integration point whose data is to be output.

### Zero-Length Elements

Zero-length elements connect two points at the same coordinate.

#### Zero-Length Element

This command is used to construct a zeroLength element object, which is defined by two nodes at the same location. The nodes are connected by multiple UniaxialMaterial (page 25) objects to represent the force-deformation relationship for the element.

element zeroLength $eleTag $iNode $jNode -mat $matTag1 $matTag2 ... -dir $dir1 $dir2 ... <-orient $x1 $x2 $x3 $yp1 $yp2 $yp3>

| $eleTag |  | unique element object tag |  |  |  |
| --- | --- | --- | --- | --- | --- |
| $iNode | $jNode | end nodes |  |  |  |
| $matTag1 $matTag2 ... |  | tags associated with previously-defined UniaxialMaterials (page 25) |  |  |  |
| $dir1 $dir2 ... |  | material directions: |  |  |  |
|  |  |  | 1,2,3 |  | translation along local x,y,z axes, respectively |
|  |  |  | 4,5,6 |  | rotation about local x,y,z axes, respectively |
| the orientation vectors can be specified for the element (optional): |  |  |  |  |  |
| $x1 $x2 $x3 |  | vector components in global coordinates defining local x-axis (vector x) |  |  |  |
| $yp1 $yp2 $yp3 |  | vector components in global coordinates defining vector yp which lies in the local x-y plane for the element: |  |  |  |
|  |  |  |  | the local z-axis is defined by the cross product between the vectors x and yp |  |

If the optional orientation vectors are not specified, the local element axes coincide with the global axes.

The valid queries to a zero-length element when creating an ElementRecorder (page 88) object are 'force,' 'deformation,' 'stiffness,' and 'material $matNum matArg1 matArg2 ...' Where $matNum is the tag associated with the material whose data is to be output.

#### Zero-Length Section Element

This command is used to construct a zeroLengthSection element object, which is defined by two nodes at the same location. The nodes are connected by a single SectionForceDeformation (page 47) object to represent the force-deformation relationship for the element.

element zeroLengthSection $eleTag $iNode $jNode $secTag <-orient $x1 $x2 $x3 $yp1 $yp2 $yp3>

| $eleTag |  | unique element object tag |  |
| --- | --- | --- | --- |
| $iNode | $jNode | end nodes |  |
| $secTag |  | tag associated with previously-defined Section (page 47) object |  |
| the orientation vectors can be specified for the element (optional): |  |  |  |
| $x1 $x2 $x3 |  | vector components in global coordinates defining local x-axis (vector x) |  |
| $yp1 $yp2 $yp3 |  | vector components in global coordinates defining vector yp which lies in the local x-y plane for the element: |  |
|  |  |  | the local z-axis is defined by the cross product between the vectors x and yp |

If the optional orientation vectors are not specified, the local element axes coincide with the global axes.

The section (page 47) force-deformation response represented by section string P acts along the element local x-axis, and the response for code Vy along the local y-axis. The other modes of section response follow from this orientation.

The valid queries to a zero-length element when creating an ElementRecorder (page 88) object are 'force,' 'deformation,' 'stiffness,' and 'section secArg1 secArg2 ....'

### Quadrilateral Elements

#### Quad Element

This command is used to construct a FourNodeQuad element object which uses a bilinear isoparametric formulation.

element quad $eleTag $iNode $jNode $kNode $lNode $thick $type $matTag <$pressure $rho $b1 $b2>

| $eleTag | unique element object tag |
| --- | --- |
| $iNode $jNode $kNode $lNode | four nodes defining element boundaries, input in counter-clockwise order around the element. |
| $thick | element thickness (constant) |
| $type | string representing material behavior. Valid options depend on the NDMaterial (page 41) object and its available material formulations. The type parameter can be either "PlaneStrain" or "PlaneStress." |
| $matTag | tag associated with previously-defined NDMaterial (page 41) object |
| $pressure | surface pressure (???? sign convention????****) |
| $rho | element mass density (per unit volume) from which a lumped element mass matrix is computed (optional, default=0.0) |
| $b1 $b2 | constant body forces defined in the isoparametric domain (optional, default=0.0) |

Consistent nodal loads are computed from the pressure and body forces.

The valid queries to a zero-length element when creating an ElementRecorder (page 88) object are 'force,'  'stiffness,' and 'material $matNum matArg1 matArg2 ...'  Where $matNum refers to the material object at the integration point corresponding to the node numbers in the isoparametric domain.

#### Shell Element

This command is used to construct a ShellMITC4 element object, which uses a bilinear isoparametric formulation in combination with a modified shear interpolation to improve thin-plate bending performance.

element ShellMITC4 $eleTag $iNode $jNode $kNode $lNode $secTag

| $eleTag | unique element object tag |
| --- | --- |
| $iNode $jNode $kNode $lNode | four nodes defining element boundaries, input in counter-clockwise order around the element. |
| $secTag | tag associated with previously-defined SectionForceDeformation (page 47) object. Typically, corresponds to some PlateFiberSection (page 59), elastic or otherwise |

Should the element be required to compute a mass matrix, a consistent translational element mass matrix is computed. Rotational-inertia terms are ignored.

The valid queries to a zero-length element when creating an ElementRecorder (page 88) object are 'force,'  'stiffness,' and 'material matArg1 matArg2 ...'

#### Bbar Plane Strain Quadrilateral Element

This command is used to construct a four-node quadrilateral element object, which uses a bilinear isoparametric formulation along with a mixed volume/pressure B-bar assumption. This element is for plane strain problems only.

element bbarQuad $eleTag $iNode $jNode $kNode $lNode $matTag

| $eleTag | unique element object tag |
| --- | --- |
| $iNode $jNode $kNode $lNode | four nodes defining element boundaries, input in counter-clockwise order around the element |
| $matTag | tag associated with previously-defined NDMaterial (page 41) object |

Should the element be required to compute a mass matrix, a consistent translational element mass matrix is computed. Rotational-inertia terms are ignored.

#### Enhanced Strain Quadrilateral Element

This command is used to construct a four-node quadrilateral element, which uses a bilinear isoparametric formulation with enhanced strain modes.

element enhancedQuad $eleTag $iNode $jNode $kNode $lNode type $matTag

| $eleTag | unique element object tag |
| --- | --- |
| $iNode $jNode $kNode $lNode | four nodes defining element boundaries, input in counter-clockwise order around the element. |
| type | string representing material behavior. Valid options depend on the NDMaterial (page 41) object and its available material formulations. The type parameter can be either "PlaneStrain" or "PlaneStress." |
| $matTag | tag associated with previously-defined NDMaterial (page 41) object |

Should the element be required to compute a mass matrix, a consistent translational element mass matrix is computed. Rotational-inertia terms are ignored.

The valid queries to a zero-length element when creating an ElementRecorder (page 88) object are 'force,'  'stiffness,' and 'material matArg1 matArg2 ...'

### Brick Elements

#### Standard Brick Element

This element is used to construct an eight-node brick element object, which uses a trilinear isoparametric formulation.

element stdBrick $eleTag $node1 $node2 $node3 $node4 $node5 $node6 $node7 $node8 $matTag

| $eleTag | unique element object tag |
| --- | --- |
| $node1 $node2 $node3 $node4 $node5 $node6 $node7 $node8 | eight nodes defining element boundaries, input order is shown in the figure |
| $matTag | tag associated with previously-defined NDMaterial (page 41) object |

Should the element be required to compute a mass matrix, a consistent translational element mass matrix is computed. Rotational-inertia terms are ignored.

- Figure 26: Node Numbering for Eight-Node Element

#### Bbar Brick Element

This command is used to construct an eight-node mixed volume/pressure brick element object, which uses a trilinear isoparametric formulation.

element bbarBrick $eleTag $node1 $node2 $node3 $node4 $node5 $node6 $node7 $node8 $matTag

| $eleTag | unique element object tag |
| --- | --- |
| $node1 $node2 $node3 $node4 $node5 $node6 $node7 $node8 | eight nodes defining element boundaries, input order is shown in the figure |
| $matTag | tag associated with previously-defined NDMaterial (page 41) object |

Should the element be required to compute a mass matrix, a consistent translational element mass matrix is computed. Rotational-inertia terms are ignored

- Figure 27: Node Numbering for Eight-Node Element

#### Eight Node Brick Element

The command is used to construct an eight-node three dimensional brick element object, which is based on tensor operation.

element Brick8N $eletag $node1 $node2 $node3 $node4 $node5 $node6 $node7 $node8 $matTag $bf1 $bf2 $bf3 $massDens

| $eletag | unique element object tag |
| --- | --- |
| $node1 $node2 $node3 $node4 $node5 $node6 $node7 $node8 | eight node coordinates, input order is shown in the figure |
| $matTag | material tag associated with previsouly-defined NDMaterial object |
| $bf1 $bf2 $bf3 | body force in the direction of global coordinates x, y and z |
| $massDens | mass density (mass/volume) |

The valid queries to a Brick8N element when creating an ElementRecorder (page 88) object are 'force,'  'stiffness,' stress', 'gausspoint' or 'plastic'. The output is given as follows:

| 'stress' | the six stress components from each Gauss points are output by the order: sigma_xx, sigma_yy, sigma_zz, sigma_xy, sigma_xz,sigma_yz |
| --- | --- |
| 'gausspoint' | the coordinates of all Gauss points are printed out |
| 'plastic' | the equivalent deviatoric plastic strain from each Gauss point is output in the same order as the coordinates are printed |

- Figure 28: Node Numbering for Eight-Node Element

#### Twenty Node Brick Element

The element is used to construct a twenty-node three dimensional element object

element Brick20N $eletag $node1 $node2 $node3 $node4 $node5 $node6 $node7 $node8 $node9 $node10 $node11 $node12 $node13  $node14 $node15 $node16 $node17 $node18 $node19 $node20 $matTag $bf1 $bf2 $bf3 $massDen

| $eletag | unique element object tag |
| --- | --- |
| $node1 $node2 $node3 $node4 $node5 $node6 $node7 $node8 $node9 $node10 $node11 $node12 $node13 $node14 $node15 $node16 $node17 $node18 $node19 $node20 | twenty node coordinates, input order is shown in the figure |
| $matTag | material tag associated with previsouly-defined NDMaterial (page 41) object |
| $bf1 $bf2 $bf3 | body force in the direction of global coordinates x, y and z |
| $massDen | mass density (mass/volume) |

The valid queries to a Brick20N element when creating an ElementRecorder (page 88) object are 'force,'  'stiffness,' stress', 'gausspoint' or 'plastic'. The output is given as follows:

| 'stress' | the six stress components from each Gauss points are output by the order: sigma_xx, sigma_yy, sigma_zz, sigma_xy, sigma_xz,sigma_yz |
| --- | --- |
| 'gausspoint' | the coordinates of all Gauss points are printed out |
| 'plastic' | the equivalent deviatoric plastic strain from each Gauss point is output in the same order as the coordinates are printed |

#### u-p-U element

This command is used to construct a u-p-U element object, which include two types: eight node element and twenty node element.

- For eight-node element:

element Brick8N_u_p_U $eleTag $node1 $node2 $node3 $node4 $node5 $node6 $node7 $node8 $matTag $bf1 $bf2 $bf3 $n $alpha $soildDens $fluidDens $k1  $k2 $k3 $K_fluid $P

- For twenty-node element:

element Brick20N_u_p_U $eleTag $node1 $node2 $node3 $node4 $node5 $node6 $node7 $node8 $node9 $node10 $node11 $node12 $node13 $node14 $node15 $node16 $node17 $node18 $node19 $node20 $matTag $bf1 $bf2 $bf3 $n $alpha $soildDens $fluidDens $k1  $k2 $k3 $K_fluid $P

| $eleTag | unique element object tag |
| --- | --- |
| $node1 $node2 $node3 $node4 $node5 $node6 $node7 $node8 .... | node coordinate (either eight or twenty), input order is shown in the figure |
| $matTag | material tag associated with previsouly-difined NDMaterial object |
| $bf1 $bf2 $bf3 | body force in the direction of global coordinates x, y and z |
| $n | porosity |
| $alpha | 1-Ks/Kt (ratio of void space =1 for soils, =0.6 for concrete...) |
| $soildDens | solid density |
| $fluidDens | fluid density |
| $k1 $k1 $k3 | coefficient of permeability in the direction of x, y and z |
| $K_fluid | fluid bulk modulus |
| $P | pressure... not used currently (set to 0.0) |

The valid queries to a Brick8N_u_p_U and Brick20N_u_p_U elements when creating an ElementRecorder (page 88) object are 'force,'  'stiffness,' stress', 'gausspoint' or 'plastic'. The output is given as follows:

| 'stress' | the six stress components from each Gauss points are output by the order: sigma_xx, sigma_yy, sigma_zz, sigma_xy, sigma_xz,sigma_yz |
| --- | --- |
| 'gausspoint' | the coordinates of all Gauss points are printed out |
| 'plastic' | the equivalent deviatoric plastic strain from each Gauss point is output in the same order as the coordinates are printed |

block Command

Chapter 9

The block command is used to generate meshes of quadrilateral or brick finite element.

The block2D (page 77) command generates meshes of quadrilateral elements in two or three dimensions. In three dimensions, a two-dimensional surface appropriate for shell analysis is generated.

The block3D (page 78) command generates three-dimensional meshes of eight-node brick solid element.

- In This Chapter block2D Command 77 block3D Command 78

### block2D Command

The block2D command generates meshes of quadrilateral elements in two or three dimensions. In three dimensions, a two-dimensional surface appropriate for shell analysis is generated.

block2d $nx $ny $e1 $n1 element (element arguments) {

1   $x1 $y1 <$z1>

2   $x2 $y2 <$z2>

3   $x3 $y3 <$z3>

4   $x4 $y4 <$z4>

<5>   <$x5> <$y5> <$z5>

<6>   <$x6> <$y6> <$z6>

<7>   <$x7> <$y7> <$z7>

<8>   <$x8> <$y8> <$z8>

<9>   <$x9> <$y9> <$z9>

}

| $nx | $ny | number of elements in the local x and y directions of the block, respectively |
| --- | --- | --- |
| $e1 | $n1 | starting element and node number for generation, respectively |
| element |  | string defining which quadrilateral element (page 69, page 70, page 68, page 69) is being used |
| (element arguments) |  | list of data parameters for element being used. This list may include, but is not limited to, a $matTag number |
| {$x1, .... $x9} {$y1, .... $y9} |  | coordinates of the block elements in two dimensions |
| {$z1, .... $z9} |  | coordinate of the block elements in third dimension (optional, default=0.0) |

Only the first four nodes (1-4) are required. Nodes 5-9 are used to generate curved meshes. The user may specify any combination of nodes 5-9, omitting some of them if desired.

EXAMPLE:

block2d $nx $ny $e1 $n1 element (element arguments) {

1   $x1 $y1 <$z1>

2   $x2 $y2 <$z2>

3   $x3 $y3 <$z3>

4   $x4 $y4 <$z4>

<5>   <$x5> <$y5> <$z5>

<6>   <$x6> <$y6> <$z6>

<7>   <$x7> <$y7> <$z7>

<8>   <$x8> <$y8> <$z8>

<9>   <$x9> <$y9> <$z9>

}

- Figure 29: Node Numbering for Nine-Node block2D

### block3D Command

The block3D command generates three-dimensional meshes of eight-node brick solid element.

block3d $nx $ny $nz $e1 $n1 element elementArgs {

1   $x1 $y1 $z1

2   $x2 $y2 $z2

3   $x3 $y3 $z3

4   $x4 $y4 $z4

5   $x5 $y5 $z5

6   $x6 $y6 $z6

7   $x7 $y7 $z7

8   $x8 $y8 $z8

<9>   <$x9> <$y9> <$z9>

...

<27> <$x27> <$y27> <$z27>

}

| $nx $ny $nz | number of elements in the local x,y and z-direction of the block |
| --- | --- |
| $e1 | starting element number for generation |
| $n1 | starting node number for generation |
| element | define which brick element (page 71, page 70) is being used |
| elementArgs | list of data parameters for element being used. This list may include, but is not limited to, a $matTag number |
| {$x1, .... $x27} {$y1, .... $y27} {$z1, .... $z27} | coordinates of the block elements in three dimensions. |

Only the first eight nodes (1-8) are required. Nodes 9-27 are used to generate curved meshes. The user may specify any combination of nodes 9-27, omitting some of them if desired.

region Command

Chapter 10

The region command is used to label a group of nodes and elements. This command is also used to assign rayleigh damping parameters to the nodes and elements in this region.

region $regTag <-ele ($ele1 $ele2 ...)>  <-eleRange $startEle $endEle> <-ele all> <-node ($node1 $node2 ...)>  <-nodeRange $startNode $endNode> <-node all>

The region is specified by either elements or nodes, not both. If elements are defined, the region includes these elements and the all connected nodes. If nodes are specified, the region includes these nodes and all elements whose external nodes are prescribed.

| $regTag | unique integer tag |
| --- | --- |
| $ele1 $ele2 ... | tags of elements -- selected elements in domain (optional, default: omitted) |
| $startEle $endEle | tag for start and end elements -- range of selected elements in domain (optional, default: all) |
| all | all elements in domain (optional & default) |

OR:

| $regTag | unique integer tag |
| --- | --- |
| $node1 $node2 ... | node tags -- select nodes in domain (optional, default: all) |
| $startNode $endNode | tag for start and end nodes -- range of nodes in domain (optional, default: all) |
| all | all nodes in domain (optional & default) |

NOTE: a region is defined by either nodes or elements, not both.

EXAMPLE:

region 1 -ele 1 5 -eleRange 10 15

region 2 -node 2 4 6 -nodeRange 9 12

Geometric Transformation Command

Chapter 11

The geometric-transformation command (geomTransf) is used to construct a coordinate-transformation (CrdTransf) object, which transforms beam element stiffness and resisting force from the basic system to the global-coordinate system. The command has at least one argument, the transformation type. Each type is outlined below.

- In This Chapter Linear Transformation 81 P-Delta Transformation 83 Corotational Transformation 84

### Linear Transformation

This command is used to construct a linear coordinate transformation (LinearCrdTransf) object, which performs a linear geometric transformation of beam stiffness and resisting force from the basic system to the global-coordinate system.

For a two-dimensional problem:

geomTransf Linear $transfTag <-jntOffset $dXi $dYi $dXj $dYj>

For a three-dimensional problem:

geomTransf Linear $transfTag $vecxzX $vecxzY $vecxzZ <-jntOffset $dXi $dYi $dZi $dXj $dYj $dZj>

| $transfTag | unique identifier for CrdTransf object |
| --- | --- |
| $vecxzX $vecxzY $vecxzZ | X, Y, and Z components of vecxz, the vector used to define the local x-z plane of the local-coordinate system. The local y-axis is defined by taking the cross product of the x-axis and the vecxz vector. These components are specified in the global-coordinate system X,Y,Z and define a vector that is in a plane parallel to the x-z plane of the local-coordinate system. These items need to be specified for the three-dimensional problem. |
| $dXi $dYi $dZi | joint offset values -- absolute offsets specified with respect to the global coordinate system for element-end node i (the number of arguments depends on the dimensions of the current model) (optional) |
| $dXj $dYj $dZj | joint offset values -- absolute offsets specified with respect to the global coordinate system for element-end node j (the number of arguments depends on the dimensions of the current model) (optional) |

The element coordinate system is specified as follows:

The x-axis is the axis connecting the two element nodes; the y- and z-axes are then defined using a vector that lies on a plane parallel to the local x-z plane -- vecxz. The y-axis is defined by taking the cross product of the x-axis and the vecxz vector. The section is attached to the element such that the y-z coordinate system used to specify the section corresponds to the y-z axes of the element.

- Figure 30: Definition of the Local Coordinate System

- Figure 31: Definition of Rigid Joint Offset (note: check sign of dXi,etc components)

### P-Delta Transformation

This command is used to construct the P-Delta Coordinate Transformation (PDeltaCrdTransf) object, which performs a linear geometric transformation of beam stiffness and resisting force from the basic system to the global coordinate system, considering second-order P-Delta effects.

For a two-dimensional problem:

geomTransf PDelta $transfTag <-jntOffset $dXi $dYi $dXj $dYj>

For a three-dimensional problem:

geomTransf PDelta $transfTag $vecxzX $vecxzY $vecxzZ <-jntOffset $dXi $dYi $dZi $dXj $dYj $dZj>

The element coordinate system and joint offset values are specified as in the Linear transformation (page 81).

| $transfTag | unique identifier for CrdTransf object |
| --- | --- |
| $vecxzX $vecxzY $vecxzZ | X, Y, and Z components of vecxz, the vector used to define the local x-z plane of the local-coordinate system. (These items need to be specified for the three-dimensional problem.) These components are specified in the global-coordinate system X,Y,Z and define a vector that is in a plane parallel to the x-z plane of the local-coordinate system. These items need to be specified for the three-dimensional problem. |
| $dXi $dYi $dZi | joint offset values -- absolute offsets specified with respect to the global coordinate system for element-end node i (the number of arguments depends on the dimensions of the current model (page 17)) (optional) |
| $dXj $dYj $dZj | joint offset values -- absolute offsets specified with respect to the global coordinate system for element-end node j (the number of arguments depends on the dimensions of the current model (page 17)) (optional) |

### Corotational Transformation

This command is used to construct the Corotational Coordinate Transformation (CorotCrdTransf) object, which performs an exact geometric transformation of beam stiffness and resisting force from the basic system to the global coordinate system.

For a two-dimensional problem:

geomTransf Corotational $transfTag <-jntOffset $dXi $dYi $dXj $dYj>

For a three-dimensional problem:

geomTransf Corotational $transfTag $vecxzX $vecxzY $vecxzZ <-jntOffset $dXi $dYi $dZi $dXj $dYj $dZj>

NOTE: The Corotational transformation is only available with the Win32 version of OpenSees (see http://opensees.berkeley.edu/OpenSees/binaries.html").

The element coordinate system and joint offset values are specified as in the Linear transformation (page 81).

| $transfTag | unique identifier for CrdTransf object |
| --- | --- |
| $vecxzX $vecxzY $vecxzZ | X, Y, and Z components of vecxz, the vector used to define the local x-z plane of the local-coordinate system. (These items need to be specified for the three-dimensional problem.) These components are specified in the global-coordinate system X,Y,Z and define a vector that is in a plane parallel to the x-z plane of the local-coordinate system. |
| $dXi $dYi $dZi | joint offset values -- absolute offsets specified with respect to the global coordinate system for element-end node i (the number of arguments depends on the dimensions of the current model (page 17)) (optional) |
| $dXj $dYj $dZj | joint offset values -- absolute offsets specified with respect to the global coordinate system for element-end node j (the number of arguments depends on the dimensions of the current model (page 17)) (optional) |

recorder Command

Chapter 12

The recorder command is used to construct a Recorder object, which is used to monitor items of interest to the analyst at each commit(). The format of the recorder command is:

recorder recorderType <arguments>

where the arguments correspond to the recorderType.

Valid strings for the recorderType are presented in this chapter, and given below.

- In This Chapter Node Recorder 85 EnvelopeNode Recorder 86 MaxNodeDisp Recorder 87 Drift Recorder 88 Element Recorder 88 EnvelopeElement Recorder 90 Display Recorder 91 Plot Recorder 92 playback Command 92

### Node Recorder

The Node type records the displacement, velocity, acceleration and incremental displacement at the nodes (translational & rotational)

recorder Node <-file $fileName> <-time> <-node ($node1 $node2 ...)>  <-nodeRange $startNode $endNode> <-region $RegionTag> <-node all> -dof ($dof1 $dof2 ...) $respType

| $fileName | file where results are stored. Each line of the file contains the result for a committed state of the domain (optional, default: screen output) |  |
| --- | --- | --- |
| -time | this argument will place the pseudo time of the as the first entry in the line. (optional, default: omitted) |  |
| $node1 $node2 ... | tags nodes where response is being recorded -- select nodes in domain (optional, default: all) |  |
| $startNode $endNode | tag for start and end nodes where response is being recorded -- range of nodes in domain (optional, default: all) |  |
| $RegionTag | tag for previously-defined selection of nodes defined using the Region command. (optional) |  |
| all | where response is being recorded -- all nodes in domain (optional & default) |  |
| $dof1 $dof2 ... | degrees of freedom of response being recorded. Valid range is from 1 through ndf (page 17) , the number of nodal degrees-of-freedom. |  |
| $respType | defines response type to be recorded. The following response types are available: |  |
|  | disp | displacement |
|  | vel | velocity |
|  | accel | acceleration |
|  | incrDisp | incremental displacement |

NOTE: $respType must be the last argument in this command.

Example:

recorder Node -file node.out –time –node 1 5 -nodeRange 10 25 -dof 2 disp

### EnvelopeNode Recorder

The Node type records the envelope of displacement, velocity, acceleration and incremental displacement at the nodes (translational & rotational). The envelope consists of the following: minimum, maximum and maximum absolute value of specified response type.

recorder EnvelopeNode <-file $fileName> <-time> <-node ($node1 $node2 ...)>  <-nodeRange $startNode $endNode> <-region $RegionTag> <-node all> -dof ($dof1 $dof2 ...) $respType

| $fileName | file where results are stored. Each line of the file contains the result for a committed state of the domain (optional, default: screen output) |  |
| --- | --- | --- |
| -time | this argument will place the pseudo time of the as the first entry in the line. (optional, default: omitted) |  |
| $node1 $node2 ... | where response is being recorded -- select nodes (optional, default: all) |  |
| $startNode $endNode | tag for start and end where response is being recorded (optional, default: all) |  |
| $RegionTag | tag for previously-defined selection of nodes defined using the Region command. (optional) |  |
| all | where response is being recorded -- all nodes in domain (optional & default) |  |
| $dof1 $dof2 ... | degrees of freedom of response being recorded. Valid range is from 1 through ndf (page 17) , the number of nodal degrees-of-freedom. |  |
| $respType | defines response type to be recorded. The following response types are available: |  |
|  | disp | displacement |
|  | vel | velocity |
|  | accel | acceleration |
|  | incrDisp | incremental displacement |

NOTE: $respType must be the last argument in this command.

Example:

recorder EnvelopeNode -file EnvelopeNode.out –time –node 1 5 -nodeRange 10 25 -dof 2 disp

### MaxNodeDisp Recorder

The MaxNodeDisp type records the values of the maximum absolute values of the displacement in the prescribed direction of a prescribed set of nodes

recorder MaxNodeDisp $dof $node1 $node2 ...

| $dof | displacement degree-of-freedom direction. Valid range is from 1 through ndf (page 17), the number of nodal degrees-of-freedom. |
| --- | --- |
| $node1 $node2 ... | nodes (page 18) where maximum displacement is being recorded |

### Drift Recorder

The Drift type records the displacement drift between two nodes. The drift is taken as the ratio between the prescribed relative displacement and the specified distance between the nodes.

recorder Drift $fileName <-time> $node1 $node2 $dof $perpDirn

| $fileName | file where results are stored. Each line of the file contains the result for a committed state of the domain |
| --- | --- |
| -time | this argument will place the pseudo time of the as the first entry in the line. (optional, default: omitted) |
| $node1 $node2 | the two for which drift is recorded |
| $dof | nodal degree of freedom to monitor for drift Valid range is from 1 through , the number of nodal degrees-of-freedom. ??? does rotation count??? |
| $perpDirn | perpendicular global direction from which length is determined to compute drift (1 = X, 2 = Y, 3 = Z) ??????? |

Example:

recorder Drift drift.out –time 2 4 1 2

### Element Recorder

The Element type records the response of a number of elements. The response recorded is element-dependent and depends on the arguments which are passed to the setResponse() element method.

recorder Element <-file $fileName> <-time> <-ele ($ele1 $ele2 ...)>  <-eleRange $startEle $endEle> <-region $regTag> <-ele all> ($arg1 $arg2 ...)

| $fileName | file where results are stored. Each line of the file contains the result for a committed state of the domain (optional, default: screen output) |
| --- | --- |
| -time | this argument will place the pseudo time of the as the first entry in the line. (optional, default: omitted) |
| $ele1 $ele2 ... | tags of elements whose response is being recorded -- selected elements in domain (optional, default: omitted) |
| $startEle $endEle | tag for start and end elements whose response is being recorded -- range of selected elements in domain (optional, default: all) |
| $regTag | previously-defined tag of region of elements whose response is being recorded -- region of elements in domain (optional) |
| all | elements whose response is being recorded -- all elements in domain (optional & default) |
| $arg1 $arg2 ... | arguments which are passed to the setResponse() element method |

The setResponse() element method is dependent on the element type, and is described with the element Command (page 60).

- Beam-Column Elements (page 64, page 65, page 62, page 63)	:

Common to all beam-column elements:

globalForce – element resisting force in global coordinates (does not include inertial forces)

example:

recorder Element -file ele1global.out -time -ele 1 globalForce

localForce – element resisting force in local coordinates (does not include inertial forces)

example:

recorder Element -file ele1local.out -time -ele 1 localForce

- Sections: (page 47)

section $secNum – request response quantities from a specific section along the element length,

$secNum refers to the integration point whose data is to be output

force – section forces

example: recorder Element -file ele1sec1Force.out –time -ele 1 section 1 force

deformation – section deformations

example: recorder Element -file ele1sec1Force.out –time -ele 1 section 1  deformation

stiffness – section stiffness

example: recorder Element -file ele1sec1Force.out –time -ele 1 section 1 stiffness

stressStrain – record  stress-strain response.

example: recorder Element -file ele1sec1Force.out –time -ele 1  section 1 fiber $y $z stressStrain

|  | $y | local y coordinate of fiber to be monitored* |
| --- | --- | --- |
|  | $z | local z coordinate of fiber to be monitored* |

*NOTE: The recorder object will search for the fiber closest to the location ($y,$z) on the section and record its stress-strain response

### EnvelopeElement Recorder

The Element type records the response of a number of elements. The response recorded is element-dependent and depends on the arguments which are passed to the setResponse() element method. The envelope consists of the following: minimum, maximum and maximum absolute value of specified response type.

recorder EnvelopeElement <-file $fileName> <-time> <-ele ($ele1 $ele2 ...)>  <-eleRange $startele $endele> <-ele all> <-region $regTag> ($arg1 $arg2 ...)

| $fileName | file where results are stored. Each line of the file contains the result for a committed state of the domain (optional, default: screen output) |
| --- | --- |
| -time | this argument will place the pseudo time of the as the first entry in the line. (optional, default: omitted) |
| $eleID1 $eleID2 ... | tags of elements whose response is being recorded -- selected elements in domain (page 14) (optional, default: omitted) |
| $startele $endele | tag for start and end elements whose response is being recorded -- selected elements in domain (page 14) (optional, default: all) |
| all | elements whose response is being recorded -- all elements in domain (page 14) (optional & default) |
| $regTag | tag of region of elements whose response is being recorded -- region of elements in domain (page 14) (optional) |
| $arg1 $arg2 ... | arguments which are passed to the setResponse() element method |

The setResponse() element method is dependent on the element type, and is described with the element Command (page 60).

- Beam-Column Elements (page 64, page 65, page 62, page 63)	:

Common to all beam-column elements:

globalForce – element resisting force in global coordinates (does not include inertial forces)

example:

recorder EnvelopeElement -file ele1global.out -time -ele 1 globalForce

localForce – element resisting force in local coordinates (does not include inertial forces)

example:

recorder EnvelopeElement -file ele1local.out -time -ele 1 localForce

- Sections: (page 47)

section $secNum – request response quantities from a specific section along the element length,

$secNum refers to the integration point whose data is to be output

force – section forces

example: recorder EnvelopeElement -file ele1sec1Force.out –time -ele 1 section 1 force

deformation – section deformations

example: recorder EnvelopeElement -file ele1sec1Force.out –time -ele 1 section 1  deformation

stiffness – section stiffness

example: recorder EnvelopeElement -file ele1sec1Force.out –time -ele 1 section 1 stiffness

stressStrain – record  stress-strain response.

example: recorder EnvelopeElement -file ele1sec1Force.out –time -ele 1  section 1 fiber $y $z stressStrain

|  | $y | local y coordinate of fiber to be monitored* |
| --- | --- | --- |
|  | $z | local z coordinate of fiber to be monitored* |

*NOTE: The recorder object will search for the fiber closest to the location ($y,$z) on the section and record its stress-strain response

### Display Recorder

This recorder opens a graphical window for displaying of graphical information.

recorder display $windowTitle $xLoc $yLoc $xPixels $yPixels <-file $fileName>

| $windowTitle | title of graphical window |
| --- | --- |
| $xLoc $yLoc | horizontal and vertical location of graphical window (upper left-most corner) |
| $xPixels $yPixels | width and height of graphical window in pixels |
| $fileName | in addition to the window display, information is sent to a file to redisplay images at a later time. (optional) |

A TclFeViewer object is constructed. This constructor adds a number of additional commands to OpenSees, similar to the construction of the BasicBuilder (page 17). These additional commands are used to define the viewing system for the image that is place on the screen. These commands are currently under review and will be presented in a future version of this document.

### Plot Recorder

This recorder type opens a graphical window for the plotting of the contents of the prescribed file

recorder plot $fileName $windowTitle $xLoc $yLoc $xPixels $yPixels -columns $xCol0 $yCol0 <-columns $xCol1 $yCol1><-columns $xCol2 $yCol2> ...

| $fileName | source file of plotted data |
| --- | --- |
| $windowTitle | title of graphical window |
| $xLoc $yLoc | horizontal and vertical location of graphical window in pixels (upper left-most corner) |
| $xPixels $yPixels | width and height of graphical window in pixels |
| $xCol0 $yCol0 | Column number to be plotted in X-axis and Y-axis, respectively. One set of columns must be defined. |
| $xCol1 $yCol1 $xCol2 $yCol2 | Additional lines may be plotted on the same graph by repeating the -columns command. These data come from the same source file. (optional) |

### playback Command

This command is used to invoke playback on all Recorder objects constructed with the recorder command (page 85).

playback $commitTag

| $commitTag | integer used to invoke the record() method (????) |
| --- | --- |

These objects are used to control the analysis.

Chapter 13

# Analysis Commands

- In This Chapter Time Series 95 pattern Command 100 analysis Command 106 constraints Command 109 integrator Command 113 algorithm Command 119 test Command 122 numberer Command 125 system Command 127 analyze Command 129 eigen Command 130 dataBase Commands 131

Time Series

Chapter 14

While there is no timeSeries command in the language, a number of commands take as the argument a list of items which defines the TimeSeries object to be constructed as part of the command, such as the LoadPattern (page 100) and groundMotion (page 103) objects.

Time series act differently depending on what type of object they are applied to:

| LoadPattern (page 100) object: |  |
| --- | --- |
|  | Load factors are applied to the loads and constraints |
| groundMotion (page 103) object: |  |
|  | Load factors are applied at the DOF in a ground motion |

The type of TimeSeries objects available are presented in this chapter.

NOTE: The TimeSeries objects are handled by the Tcl interpreter as lists. Therefore, they can be defined a-priori within quotes "" and given a variable name. EXAMPLE:

set Gaccel "Series -dt $dt -filePath $outFile -factor  $GMfatt";	# time series information

pattern UniformExcitation  2   1  -accel $Gaccel;		# create uniform excitation

- In This Chapter Constant Time Series 96 Linear Time Series 96 Rectangular Time Series 97 Sine Time Series 97 Path Time Series 98

### Constant Time Series

This command creates a ConstantSeries TimeSeries (page 95)  object and associates it to the LoadPattern (page 100) object being constructed.

Constant <-factor $cFactor>

| $cFactor | load-factor coefficient. (optional. default = 1.0) |
| --- | --- |

The load factor to be applied to the loads and constraints in the LoadPattern object is constant and equal to $cFactor.

- Figure 32: Constant Time Series

### Linear Time Series

This command creates a LinearSeries TimeSeries (page 95) object and associates it to the LoadPattern (page 100) or groundMotion (page 103) object being constructed.

Linear <-factor $cFactor>

| $cFactor | load-factor coefficient. (optional. default = 1.0) |
| --- | --- |

The load factor to be applied to the loads and constraints in the LoadPattern or groundMotion object is equal to $cFactor* time

- Figure 33: Linear Time Series

### Rectangular Time Series

This command creates a RectangularSeries TimeSeries (page 95) object and associates it to the LoadPattern (page 100) object being constructed.

Rectangular $tStart $tFinish <-factor $cFactor>

| $tStart | start time when the load factor is applied |
| --- | --- |
| $tFinish | end time when the load factor is applied |
| $cFactor | load-factor coefficient. (optional. default = 1.0) |

The load factor to be applied to the loads and constraints in the LoadPattern object is constant and equal to $cFactor during the domain time from $tStart to $tFinish

- Figure 34: Rectangular Time Series

### Sine Time Series

This command creates a TrigSeries TimeSeries (page 95) object and associates it to the LoadPattern (page 100) object being constructed.

Sine $tStart $tFinish $period <-shift $shift> <-factor $cFactor>

| $tStart | start time when the load factor is applied |
| --- | --- |
| $tFinish | end time when the load factor is applied |
| $period | characteristic period of sine wave |
| $shift | phase shift (radians) (optional. default = 0.0) |
| $cFactor | load-factor coefficient. (optional. default = 1.0) |

The load factor applied to the loads and constraints in the LoadPattern object is equal to:

- Figure 35: Sine Time Series

### Path Time Series

This command associates a TimeSeries (page 95) object of the type PathSeries or PathTimeSeries (if the increment is not constant) to a LoadPattern (page 100) object.

There are many ways to specify the load path.

For a load path where the values are specified at constant time intervals:

Series -dt $dt -values {list_of_values} <-factor $cFactor>

where the values are specified in a list included in the command

Series -dt $dt -filePath $fileName <-factor $cFactor>

where the values are taken from a file specified by $fileName

For a load path where the values are specified at non-constant time intervals:

Series -time {list_of_times} -values {list_of_values} <-factor $cFactor>

where both time and values are specified in a list included in the command

Series -fileTime $fileName1 -filePath $fileName2 <-factor $cFactor>

where both time and values are taken from a file specified by $fileName1 (for the time data) and $fileName2 (for the values data)

| $cFactor | load-factor coefficient. (optional. default = 1.0) |
| --- | --- |

The load factor to be applied to the loads and constraints in the LoadPattern object is  equal to $cFactor*(user-defined series)

pattern Command

Chapter 15

The pattern command is used to construct a LoadPattern object, its associated with the TimeSeries (page 95) object and the Load (page 101) and Constraint (page 109) objects for the pattern.

- In This Chapter plain Pattern 100 UniformExcitation Pattern 102 MultipleSupport Pattern 103

### plain Pattern

This command is used to construct an ordinary LoadPattern (page 100) object in the Domain (page 14).

pattern Plain $patternTag (TimeSeriesType arguments) {

load (load-command arguments)

sp (sp-command arguments)

eleLoad (eleLoad-command arguments)

}

| $patternTag | unique pattern object tag |
| --- | --- |
| TimeSeriesType arguments | list which is parsed to construct the TimeSeries (page 95) object associated with the LoadPattern object. |
| load ... | list of commands to construct nodal loads -- the NodalLoad (page 101) object |
| sp ... | list of commands to construct single-point constraints -- the SP_Constraint (page 101) object |
| eleLoad ... | list of commands to construct element loads -- the eleLoad (page 102) object |

NOTE: The TimeSeries object is handled by the Tcl interpreter as a list and can be defined a-priori and given a variable name.

EXAMPLE

pattern Plain 1 Linear  {  ;		# define LoadPattern 1. impose load in a linear manner

load 3 100 0. 0. 0. 0. 20.;		# apply force and moment at node 3

}

#### load Command

This command is used to construct a NodalLoad object.

load $nodeTag (ndf $LoadValues)

The nodal load is added to the LoadPattern being defined in the enclosing scope of the pattern command.

| $nodeTag | node on which loads act |
| --- | --- |
| $LoadValues | load values that are to be applied to the node. Valid range is from 1 through ndf (page 17), the number of nodal degrees-of-freedom. |

EXAMPLE

load 3 100 0. 0. 0. 0. 20.;	# apply force Fx=100 and moment Mz=20 at node 3

#### sp Command

This command is used to construct a single-point non-homogeneous constraint (SP_Constraint) object.

sp $nodeTag $DOFtag $DOFvalue

| $nodeTag | node on which the single-point constraint acts |
| --- | --- |
| $DOFtag | degree-of-freedom at the node being constrained. Valid range is from 1 through ndf (page 17), the number of nodal degrees-of-freedom. |
| $DOFvalue | reference value of the constraint to be applied to the DOF at the node. |

EXAMPLE

load 3 0.1 0. 0. 0. 0. 0.;	# impose displacement Dx=0.1 at node 3

#### eleLoad Command

i NEED information from Michael!!

### UniformExcitation Pattern

This command is used to construct a UniformExcitation load pattern object.

pattern UniformExcitation $patternTag $dir -accel (TimeSeriesType arguments) <-vel0 $ver0>

| $patternTag | unique pattern object tag |
| --- | --- |
| $dir | direction of excitation (1, 2, or 3) used in formulating the inertial loads for the transient analysis. |
| TimeSeriesType arguments | TimeSeries (page 95) object associated with the acceleration record used in determining the inertial loads. |
| $vel0 | initial velocity to be assigned to each node (optional, default: zero) |

NOTE: The TimeSeries object is handled by the Tcl interpreter as a list and can be defined a-priori and given a variable name.

EXAMPLE:

set Gaccel "Series -dt $dt -filePath $outFile -factor  $GMfatt";	# time series information

pattern UniformExcitation  2   1  -accel $Gaccel;			# create uniform excitation with IDtag 2 in direction 1

### MultipleSupport Pattern

This command is used to construct a MultipleSupportExcitation load pattern object.

pattern MultipleSupport $patternTag {

groundMotion (groundMotion-command arguments)

imposedMotion (imposedMotion-command arguments)

}

| $patternTag | unique pattern object tag |
| --- | --- |
| groundMotion ... | list of commands to construct the GroundMotions (page 103) object that is then added to the object to define the multiple-support excitation that is being imposed on the model |
| imposedMotion ... | list of commands to construct the ImposedSupportSP (page 105) constraint object that is then added to the object to define the multiple-support excitation that is being imposed on the model |

#### groundMotion Command

The groundMotion command is used to construct a GroundMotion object used by the ImposedMotionSP (page 105) constraints in a MultipleSupportExcitation (page 103) object.

##### Plain GroundMotion

This command is used to construct a plain GroundMotion object. Each GroundMotion object is associated with a number of TimeSeries (page 95) objects, which define the acceleration, velocity and displacement records.

groundMotion $gMotionTag Plain <-accel (accelSeriesType accelArgs)> <-vel (velSeriesType velArgs)> <-disp (dispSeriesType dispArgs)> <-int (IntegratorType intArgs)>

| $gMotionTag |  | unique GroundMotion (page 103) object tag |
| --- | --- | --- |
| <-accel (accelSeriesType accelArgs)> |  |  |
|  | TimeSeries (page 95) objects defining the acceleration record (optional). |  |
| <-vel (velSeriesType velArgs)> |  |  |
|  | TimeSeries (page 95) objects defining the velocity record (optional) |  |
| <-disp (dispSeriesType dispArgs)> |  |  |
|  | TimeSeries (page 95) objects defining the displacement record (optional) |  |
| <-int (IntegratorType intArgs)> |  |  |
|  | If only the acceleration record is specified, the user has the option of specifying the TimeSeriesIntegrator (page 113) that is to be used to integrate the acceleration record to determine the velocity and displacement record (optional, default: Trapezoidal) |  |

NOTE:  The TimeSeries object is handled by the Tcl interpreter as a list and can be defined a-priori and given a variable name.

NOTE:  Any combination of the acceleration, velocity and displacement time-series can be specified.

##### Interpolated GroundMotion

This command is used to construct an InterpolatedGroundMotion object.

groundMotion $gMotionTag Interpolated $gmTag1 $gmTag2 ... -fact $fact1 $fact2 ...

| $gMotionTag | unique GroundMotion (page 103) object tag |
| --- | --- |
| $gmTag1 $gmTag2 ... | ground motions which have already been added to the MultipleSupportExcitation (page 103) object. |
| $fact1 $fact2 ... | factors that are used in the interpolation of the ground motions to determine the ground motion for this InterpolatedGroundMotion object. |

#### imposedMotion Command

This command is used to construct an ImposedMotionSP constraint which is used to enforce the response of a dof at a node in the model. The response enforced at the node at any give time is obtained from the GroundMotion (page 103) object associated with the constraint.

imposedMotion $nodeTag $dirn $gMotionTag

| $nodeTag | node where response is enforced |
| --- | --- |
| $dirn | dof of enforced response Valid range is from 1 through ndf (page 17), the number of nodal degrees-of-freedom. |
| $gMotionTag | pre-defined GroundMotion (page 103) object tag |

NOTE: The GroundMotion (page 103) object must be added to the MultipeSupportExcitation (page 103) pattern before the ImposedMotionSP constraint.

FMK: ADD TIME-SERIES INTEGRATORS

analysis Command

Chapter 16

This command is used to construct the Analysis object (page 15).

All currently-available analysis objects employ incremental solution strategies.

- In This Chapter Static Analysis 106 Transient Analysis 107 VariableTransient Analysis 107

### Static Analysis

This command is used to construct a StaticAnalysis object.

analysis Static

This analysis object is constructed with the component objects previously created by the analyst. If none has been created, default objects are constructed and used:

| Component Object | Default object |
| --- | --- |
| SolutionAlgorithm (page 119), StaticIntegrator (page 114, page 113, page 114, page 115) | NewtonRaphson (page 119) EquiSolnAlgo with a CTestNormUnbalance (page 122) with a tolerance of 1e-6 and a maximum of 25 iterations |
| ConstraintHandler (page 109) | PlainHandler (page 110) ConstraintHandler |
| DOF_Numberer (page 125) | RCM (page 125) DOF_Numberer |
| LinearSOE (page 127), LinearSolver (page 127) | profiled symmetric positive definite (page 128) LinearSOE and Linear Solver |
| Integrator | LoadControl StaticIntegrator (page 114) with a constant load increment of 1.0 |

### Transient Analysis

This command is used to construct a DirectIntegrationAnalysis object.

analysis Transient

This analysis object is constructed with the component objects previously created by the analyst. If none has been created, default objects are constructed and used:

| Component Object | Default object |
| --- | --- |
| SolutionAlgorithm (page 119), TransientIntegrator (page 116, page 118, page 113, page 117) | NewtonRaphson (page 119) EquiSolnAlgo with a CTestNormUnbalance (page 122) with a tolerance of 1e-6 and a maximum of 25 iterations |
| ConstraintHandler (page 109) | PlainHandler (page 110) ConstraintHandler |
| DOF_Numberer (page 125) | RCM (page 125) DOF_Numberer |
| LinearSOE (page 127), LinearSolver (page 127) | profiled symmetric positive definite (page 128) LinearSOE and Linear Solver |
| Integrator | Newmark TransientIntegrator (page 117) with =0.5 and =0.25 |

### VariableTransient Analysis

This command is used to construct a VariableTimeStepDirectIntegrationAnalysis object.

analysis VariableTransient

This analysis object is constructed with the component objects previously created by the analyst. If none has been created, default objects are constructed and used:

| Component Object | Default object |
| --- | --- |
| SolutionAlgorithm (page 119), TransientIntegrator (page 116, page 118, page 113, page 117) | NewtonRaphson (page 119) EquiSolnAlgo with a CTestNormUnbalance (page 122) with a tolerance of 1e-6 and a maximum of 25 iterations |
| ConstraintHandler (page 109) | PlainHandler (page 110) ConstraintHandler |
| DOF_Numberer (page 125) | RCM (page 125) DOF_Numberer |
| LinearSOE (page 127), LinearSolver (page 127) | profiled symmetric positive definite (page 128) LinearSOE and Linear Solver |
| Integrator | Newmark TransientIntegrator (page 117) with =0.5 and =0.25 |

constraints Command

Chapter 17

This command is used to construct the ConstraintHandler object. Constraints enforce a relationship between degrees-of-freedom. The ConstraintHandler object determines how the constraint equations are enforced in the analysis.

The constraint equations may take the following forms:

U represents the degrees of freedom. The matrix C contains constants, and the matrix I is the identity matrix. The matrix T is the transformation matrix, defined above. The subscripts indicate the following: r =retained, c =condensed.

Of the different methods, "the Lagrange multiplier method is more attractive than the transformation method if there are few constraint equations that couple many DOF. However, Lagrange multipliers are active at the structure level, but transformation equations can be applied at either the structure level or element by element. The latter has the appeal of disposing of constraints at an early stage, when the matrices are small and manageable". (Cook)

"In comparison with Lagrange multipliers, penalty functions have the advantage of introducing no new variables. However, the penalty matrix may significantly increase the bandwidth of the structural equations, depending on how DOF are numbered and what DOF are coupled by the constraint equations. Penalty functions have the disadvantage that penalty numbers must be chosen in an allowable range: large enough to be effective but not so large as to provoke numerical difficulties". (Cook)

- In This Chapter Plain Constraints 110 Penalty Method 111 Lagrange Multipliers 111 Transformation Method 112

### Plain Constraints

This command creates a PlainHandler which is only capable of enforcing homogeneous single-point constraints. If other types of constraints exist in the domain, a different constraint handler must be specified.

constraints Plain

### Penalty Method

This command is used to construct a PenaltyConstraintHandler which will cause the constraints to be enforced using a penalty method. The penalty method consists of adding large numbers to the stiffness matrix and the restoring-force vectors to impose a prescribed zero or nonzero DOF.

constraints Penalty $alphaSP $alphaMP

| $alphaSP | factor used when adding the single-point constraint into the system of equations |
| --- | --- |
| $alphaMP | factor used when adding the multi-point constraint into the system of equations |

In this method the potential-energy equation which makes up the system of equations is augmented by a penalty function {t}T[]{t}/2

where [] is a diagonal matrix of "penalty numbers". The resulting system of equations is of the form:

Where CTC can be called the penalty matrix. C and Q are matrices containing constants, K is the stiffness matrix, U represents the DOF and R the restoring forces. If =0 the constraints are ignored. As grows, U changes in such a way that the constraint equations are more nearly satisfied. In this case, however, the analysis becomes error prone, as the system represents a stiff region supported by a flexible region.

"Guideline for choice of : If computer words carry approximately p decimal digits, experience has shown that should not exceed 10p/2". (Cook)

### Lagrange Multipliers

This command is used to construct a LagrangeConstraintHandler which will cause the constraints to be enforced using the method of Lagrange multipliers. From  Cook: "Lagrange's method of undetermined multipliers is used to find the maximum or minimum of a function whose variables are not independent but have some prescribed relation. In structural mechanics the function is the potential energy and the variables are the DOF".

constraints Lagrange <$alphaSP> <$alphaMP>

| $alphaSP | factor used when adding the single-point constraint into the system of equations (optional, default=1.0) |
| --- | --- |
| $alphaMP | factor used when adding the multi-point constraint into the system of equations (optional, default=1.0) |

NOTE: Values for $alphaSP and $alphaMP other than 1.0 are permitted to offset numerical roundoff problems.

NOTE: The system of equations is not positive defined due to the introduction of zeroes on the diagonal by the constraint equations:

### Transformation Method

This command is used to construct a TransofrmationConstraintHandler which will cause the constraints to be enforced using the transformation method.

constraints Transformation

NOTE: With the current implementation, a retained node in an MP_Constraint cannot also be specified as being a constrained node in another MP_Constraint.

The constraint equations takes the following form:

integrator Command

Chapter 18

This command is used to construct the Integrator object. The Integrator object determines the meaning of the terms in the system of equation object (???).

The Integrator object is used for the following:

- determine the predictive step for time t+dt

- specify the tangent matrix and residual vector at any iteration

- determine the corrective step based on the displacement increment dU

The system of nonlinear equations is of the form:

| Static analysis: |  |
| --- | --- |
| Transient analysis: |  |

The type of integrator used in the analysis is dependent on whether it is a static analysis (page 106) or transient analysis (page 107):

| STATIC ANALYSIS* |  |
| --- | --- |
| - LoadControl (page 114) |  |
| - DisplacementControl (page 114) |  |
| - MinUnbalDispNorm (page 115) |  |
| - ArcLength (page 116) |  |
| TRANSIENT ANALYSIS |  |
| - Newmark (page 117) |  |
| - Hilbert-Hughes-Taylor Method (HHT (page 118)) |  |

*NOTE: static integrators should only be used with a Linear TimeSeries (page 96) object with a factor of 1.0.

- In This Chapter Load Control 114 Displacement Control 114 Minimum Unbalanced Displacement Norm 115 Arc-Length Control 116 Newmark Method 117 Hilbert-Hughes-Taylor 118

### Load Control

This command is used to construct a StaticIntegrator object of type LoadControl

integrator LoadControl $dLambda1 <$Jd $minLambda $maxLambda>

| $dLambda1 | first load increment (pseudo-time step) in the next invocation of the analysis (page 106) command. |
| --- | --- |
| $Jd | factor relating load increment at subsequent time steps. (optional, default: 1.0) |
| $minLambda $maxLambda | arguments used to bound the load increment (optional, default: $dLambda1 for both) |

The load increment at iterations i, dLambda(i), is related to the load increment at (i-1), dLambda(i-1), and the number of iterations at (i-1), J(i-1), by the following:

dLambda(i) = dLambda(i-1)*Jd/J(i-1)

### Displacement Control

This command is used to construct a StaticIntegrator object of the type DisplacementControl.

integrator DisplacementControl $nodeTag $dofTag $dU1 <$Jd $minDu $maxDu>

| $nodeTag | node whose response controls the solution |
| --- | --- |
| $dofTag | degree-of-freedom whose response controls the solution. Valid range is from 1 through ndf (page 17), the number of nodal degrees-of-freedom. |
| $dU1 | first displacement increment (pseudo-time step) in the next invocation of the analysis command |
| $Jd | factor relating displacement increment at subsequent time steps. (optional, default: 1.0) |
| $minDu $maxDu | arguments used to bound the displacement increment (optional, default: $dU1 for both) |

The displacement increment at iterations i, dU(i), is related to the displacement increment at (i-1), dU(i-1), and the number of iterations at (i-1), J(i-1), by the following:

dU(i) = dU(i-1)*Jd/J(i-1)

### Minimum Unbalanced Displacement Norm

This command is used to construct a StaticIntegrator object of type MinUnbalDispNorm.

integrator MinUnbalDispNorm $dlambda11 <$Jd $minLambda $maxLambda>

| $dLambda11 | first load increment (pseudo-time step) at the first iteration in the next invocation of the analysis (page 106) command. |
| --- | --- |
| $Jd | factor relating first load increment at subsequent time steps. (optional, default: 1.0) |
| $minLambda $maxLambda | arguments used to bound the load increment (optional, default: $dLambda11 for both) |

The load increment at iterations i, dLambda1(i), is related to the load increment at (i-1), dLambda1(i-1), and the number of iterations at (i-1), J(i-1), by the following:

dLambda1(i) = dLambda1(i-1)*Jd/J(i-1)

### Arc-Length Control

This command is used to construct a StaticIntegrator object of type ArcLength. Arc-length methods are used to enable the solution algorithm to pass limit points, such as maximum and minimum loads, and snap-through and snap-back responses. At these limit points, the stability of the numerical system is dependent on whether the analysis is performed under load or displacement control. In structural analysis, these limit points are characteristic of cracking of reinforced concrete and of buckling of shells.

integrator ArcLength $arclength $alpha

(??? is this the correct order, at the workshop Frank had it different)

| $arclength | arclength value |
| --- | --- |
| $alpha |  |

SEE FMK

The equilibrium equation can be written in the from:

the arc-length method aims to find the intersection of the above equation with s=constant, where s is the arc-length, defined by:

and

the scaling parameter  is required because the load contribution depends on the adopted scaling between the load and displacement terms.

for the arc-length methods, one should replace the differential form of the equation for ds with an incremental form:

where l is the fixed 'radius of the desired intersection.

In the arc-length method the load parameter  becomes a variable, adding one to the n displacement variables and equations.

### Newmark Method

This command is used to construct a TransientIntegrator object of type Newmark.

integrator Newmark $gamma $beta <$alphaM $betaK $betaKinit $betaKcomm>

| $gamma | Newmark parameter  |
| --- | --- |
| $beta | Newmark parameter  |
| $alphaM $betaK $betaKinit $betaKcomm | Arguments to define Rayleigh damping matrix (optional, default: zero) |

The damping matrix D is specified as a combination of stiffness and mass-proportional damping matrices:

D = $alphaM * M + $betaK * Kcurrent +$betaKinit * Kinit + $betaKcomm * KlastCommit

@

The mass and stiffness matrices are defined as:

| M | mass matrix used to calculate Rayleigh Damping |
| --- | --- |
| Kcurrent | stiffness matrix at current state determination used to calculate Rayleigh Damping |
| Kinit | stiffness matrix at initial state determination used to calculate Rayleigh Damping |
| KlastCommit | stiffness matrix at last-committed state determination used to calculate Rayleigh Damping |

### Hilbert-Hughes-Taylor

This command is used to construct a TransientIntegrator object of type HHT or HHT1.

integrator HHT $gamma <$alphaM $betaK $betaKinit $betaKcomm>

| $gamma | Newmark parameter  |
| --- | --- |
| $alphaM $betaK $betaKinit $betaKcomm | Arguments to define Rayleigh damping matrix (optional, default: zero) |

The damping matrix D is specified as a combination of stiffness and mass-proportional damping matrices:

D = $alphaM * M + $betaK * Kcurrent +$betaKinit * Kinit + $betaKcomm * KlastCommit

The mass and stiffness matrices are defined as:

| M | mass matrix |
| --- | --- |
| Kcurrent | stiffness matrix at current state determination |
| Kinit | stiffness matrix at initial state determination |
| KlastCommit | stiffness matrix at last-committed state determination |

algorithm Command

Chapter 19

This command is used to construct a SolutionAlgorithm object, which determines the sequence of steps taken to solve the non-linear equation.

- In This Chapter Linear Algorithm 119 Newton Algorithm 119 Newton with Line Search Algorithm 120 Modified Newton Algorithm 120 Krylov-Newton Algorithm 121 BFGS Algorithm 121 Broyden Algorithm 121

### Linear Algorithm

This command is used to construct a Linear algorithm object which takes one iteration to solve the system of equations.

algorithm Linear

### Newton Algorithm

This command is used to construct a NewtonRaphson algorithm object which uses the Newton-Raphson method to advance to the next time step.

algorithm Newton

NOTE: The tangent is updated at each iteration.

### Newton with Line Search Algorithm

This command is used to construct a NewtonLineSearch algorithm object which uses the Newton-Raphson method with line search to advance to the next time step.

algorithm NewtonLineSearch $ratio

| $ratio | limiting ratio between the residuals before and after the incremental update (between 0.5 and 0.8) If the ratio between the residuals before and after the incremental update is greater than the specified limiting ratio the line search algorithm developed by Crissfield (REF?) is employed to try to improve the convergence. |
| --- | --- |

### Modified Newton Algorithm

This command is used to construct a ModifiedNewton algorithm object which uses the modified Newton-Raphson method to advance to the next time step.

algorithm ModifiedNewton

NOTE: The tangent at the first iteration of the current time step is used to iterate on the next time step.

### Krylov-Newton Algorithm

This command is used to construct a KrylovNewton algorithm object which uses a modified Newton method with Krylov subspace acceleration to advance to the next time step.

algorithm KrylovNewton

The accelerator is described by Carlson and Miller in "Design and Application of a 1D GWMFE Code" from SIAM Journal of Scientific Computing (see http://epubs.siam.org/sam-bin/dbq/toclist/SISC") (Vol. 19, No. 3, pp. 728-765, May 1998).

### BFGS Algorithm

This command is used to construct a BFGS algorithm object for symmetric systems which performs successive rank-two updates of the tangent at the first iteration of the current time step.

algorithm BFGS <$count>

| $count | number of iterations within a time step until a new tangent is formed |
| --- | --- |

### Broyden Algorithm

This command is used to construct a Broyden algorithm object for general unsymmetric systems which performs successive rank-one updates of the tangent at the first iteration of the current time step.

algorithm Broyden <$count>

| $count | number of iterations within a time step until a new tangent is formed |
| --- | --- |

test Command

Chapter 20

This command is used to construct a ConvergenceTest object. Certain SolutionAlgorithm (page 119) objects require a ConvergenceTest object to determine if convergence has been achieved at the end of an iteration step. The convergence test is applied to the following equation:

The test perform the following checks:

| - Norm Unbalance |  |
| --- | --- |
| - Norm Displacement Increment |  |
| - Energy Increment |  |

- In This Chapter Norm Unbalance Test 122 Norm Displacement Increment T 123 Energy Increment Test 124

### Norm Unbalance Test

This command is used to construct a CTestNormUnbalance object which tests positive force convergence if the 2-norm of the b vector (the unbalance) in the LinearSOE (page 127) object is less than the specified tolerance.

test NormUnbalance $tol $maxNumIter <$printFlag>

| $tol | convergence tolerance |  |  |
| --- | --- | --- | --- |
| $maxNumIter | maximum number of iterations that will be performed before "failure to converge" is returned |  |  |
| $printFlag | flag used to print information on convergence (optional) |  |  |
|  |  | 0 | no print output (default) |
|  |  | 1 | print information on each step |
|  |  | 2 | print information when convergence has been achieved |
|  |  | 4 | print norm, dU and dR vectors |
|  |  | 5 | at convergence failure, carry on, print error message, but do not stop analysis |

The test performs the following check:

### Norm Displacement Increment T

This command is used to construct a CTestNormDispIncr object which tests positive force convergence if the 2-norm of the x vector (the displacement increment) in the LinearSOE (page 127) object is less than the specified tolerance.

test NormDispIncr $tol $maxNumIter <$printFlag>

| $tol | convergence tolerance |  |  |
| --- | --- | --- | --- |
| $maxNumIter | maximum number of iterations that will be performed before "failure to converge" is returned |  |  |
| $printFlag | flag used to print information on convergence (optional) |  |  |
|  |  | 0 | no print output (default) |
|  |  | 1 | print information on each step |
|  |  | 2 | print information when convergence has been achieved |
|  |  | 4 | print norm, dU and dR vectors |
|  |  | 5 | at convergence failure, carry on, print error message, but do not stop analysis |

The test performs the following check:

### Energy Increment Test

This command is used to construct a CTestEnergyIncr object which tests positive force convergence if one half of the inner-product of the x and b vectors (displacement increment and unbalance) in the LinearSOE (page 127) object is less than the specified tolerance.

test EnergyIncr $tol $maxNumIter <$printFlag>

| $tol | convergence tolerance |  |  |
| --- | --- | --- | --- |
| $maxNumIter | maximum number of iterations that will be performed before "failure to converge" is returned |  |  |
| $printFlag | flag used to print information on convergence (optional) |  |  |
|  |  | 0 | no print output (default) |
|  |  | 1 | print information on each step |
|  |  | 2 | print information when convergence has been achieved |
|  |  | 4 | print norm, dU and dR vectors |
|  |  | 5 | at convergence failure, carry on, print error message, but do not stop analysis |

The test performs the following check:

numberer Command

Chapter 21

This command is used to construct the DOF_Numberer object. The DOF_Numberer object determines the mapping between equation numbers and degrees-of-freedom -- how degrees-of-freedom are numbered.

The types of numberer objects available:

| - Plain (page 125) | nodes are assigned degrees-of-freedom arbitrarily |
| --- | --- |
| - RCM (page 125) | nodes are assigned degrees-of-freedom using the Reverse Cuthill-McKee algorithm (REF???) |

As certain system of equation and solver objects do their own mapping, i.e. SuperLU, UmfPack, Kincho's, specifying a numberer other than plain may not be needed.

- In This Chapter Plain Numberer 125 RCM Numberer 125

### Plain Numberer

This command is used to construct a PlainNumberer object.

numberer Plain

The Plain numberer assigns degrees-of-freedom to the nodes based on how the nodes are stored in the domain. Currently, the user has no control over how nodes are stored.

### RCM Numberer

This command is used to construct a RCMNumberer object.

numberer RCM

The RCM numberer uses the reverse Cuthill McKee (REF?) algorithm to number the degrees of freedom.

system Command

Chapter 22

This command is used to construct the LinearSOE and LinearSolver objects to store and solve the system of equations in the analysis.

- Figure 36: system command

- In This Chapter BandGeneral SOE 127 BandSPD SOE 128 ProfileSPD SOE 128 SparseGeneral SOE 128 UmfPack SOE 128 SparseSPD SOE 128

### BandGeneral SOE

This command is used to construct an un-symmetric banded system of equations object which will be factored and solved during the analysis using the Lapack band general solver.

system BandGeneral

### BandSPD SOE

This command is used to construct a symmetric positive definite banded system of equations object which will be factored and solved during the analysis using the Lapack band spd solver.

system BandSPD

### ProfileSPD SOE

This command is used to construct symmetric positive definite profile system of equations object which will be factored and solved during the analysis using a profile solver.

system ProfileSPD

### SparseGeneral SOE

This command is used to construct a general sparse system of equations object which will be factored and solved during the analysis using the SuperLU solver.

system SparseGeneral <-piv>

| -piv | perform partial-pivoting (optional, Default: no partial pivoting is performed) |
| --- | --- |

### UmfPack SOE

This command is used to construct a general sparse system of equations object which will be factored and solved during the analysis using the UMFPACK solver. (REF?)

system UmfPack

### SparseSPD SOE

This command is used to construct a sparse symmetric positive definite system of equations object which will be factored and solved during the analysis using a sparse solver developed at Stanford University by Kincho Law.  (REF?)

system SparseSPD

analyze Command

Chapter 23

This command is invoked on the Analysis object constructed with the analysis command (page 106).

analyze $numIncr <$dt> <$dtMin $dtMax $Jd>

| $numIncr |  | number of load steps |
| --- | --- | --- |
| $dt |  | time-step increment. Required if a transient analysis (page 107) or variable time step transient analysis (page 107) was specified. |
| $dtMin | $dtMax | minimum and maximum time steps Required if a variable time step transient analysis (page 107) was specified. |
| $Jd |  | number of iterations performed at each step Required if a variable time step transient analysis (page 107) was specified. |

This command RETURNS:

|  | 0 | successful |
| --- | --- | --- |
|  | <0 | unsuccessful |

eigen Command

Chapter 24

This command is used to perform a generalized eigenvalue problem to determine a specified number of eigenvalues and eigenvectors.

eigen <$type> $numEigenvalues

| $type | eigen-value analysis type: |  |  |
| --- | --- | --- | --- |
|  |  | frequency | solve: K-  M |
|  |  | generalized | solve: K-  M (default) |
|  |  | standard | solve: K-  I |
| $numEigenvalues | number of first eigenvalues ( to be determined |  |  |

The eigenvectors are stored at the nodes and can be printed out using Node Recorder (page 85) or the Print command (page 133).

dataBase Commands

Chapter 25

This command is used to construct a FE_Datastore object.

Currently there is only one type of Datastore object available.

- In This Chapter FileDatastore Command 131

### FileDatastore Command

This command is used to construct the FE_Datastore object.

database $type $dbName

| $type | database type: |  |  |
| --- | --- | --- | --- |
|  |  | File | outputs database into a file |
|  |  | MySQL | creates a SQL database |
|  |  | BerkeleyDB | creates a BerkeleyDB database |
| $dbName | database name. If the database type is File, the command will save the data into a number of files, e.g. $dbName.id11 for all ID objects of size 11 that sendSelf() is invoked upon. |  |  |

The invocation of this command will add the additional commands save and restore to the OpenSees interpreter to allow users to save and restore model states.

#### save Command

This command is used to save the state of the model in the database.

save $commitTag

| $commitTag | unique identifier that can be used to restore (page 132) the state at a later time |
| --- | --- |

#### restore Command

This command is used to restore the state of the model from the information stored in the database.

restore $commitTag

| $commitTag | unique identifier used to restore the state at the model when the save (page 131) command was invoked |
| --- | --- |

These are a few additional miscellaneous command used in OpenSees

Chapter 26

# Miscellaneous Commands

- In This Chapter print Command 133 reset Command 134 wipe Command 134 wipeAnalysis Command 134 loadConst Command 135 getTime Command 135 nodeDisp Command 135 video Command 136

## print Command

This command is used to print output.

To print all objects of the domain:

print <$fileName>

To print node information:

print <$fileName> -node <-flag $flag> <$node1 $node2 ...>

To print element information:

print <$fileName> -ele <-flag $flag> <$ele1 $ele2 ...>

| $fileName | fileName for printed output (optional, Default: stderr -- screen dump) |
| --- | --- |
| $flag | integer flag to be sent to the print() method, depending on the node and element type (optional) |
| $node1 $node2 ... | node tag for selected-node output (optional) Default: all |
| $ele1 $ele2 ... | element tag for selected-element output (optional) Default: all |

## reset Command

This command is used to set the state of the domain to its original state.

reset

The command invokes revertToStart() on the Domain (page 14) object.

## wipe Command

This command is used to destroy all constructed objects.

wipe

This command is used to start over without having to exist and restart the interpreter (page 12).

## wipeAnalysis Command

This command is used to destroy all objects constructed for the analysis.

wipeAnalysis

This command is used to start a new type of analysis. This command does not destroy the elements (page 60), nodes (page 18), materials (page 41, page 25), etc. It does destroy the solution strategies: the algorithm (page 119), analysis (page 106), equation solver (page 127), constraint handler (page 109), etc.

## loadConst Command

This command is used to invoke setLoadConst()  on all LoadPattern (page 100) objects which have been created up to this point.

loadConst <-time $pseudoTime>

| -time $pseudoTime | set pseudo time in the domain to $pseudoTime (optional, default: zero) |
| --- | --- |

## getTime Command

This command returns the time in the domain.

getTime

## nodeDisp Command

Returns the displacement or rotation at specified node.

nodeDisp $nodeTag $dof

| $nodeTag | node tag |
| --- | --- |
| $dof | degree-of-freedom tag Valid range is from 1 through ndf (page 17), the number of nodal degrees-of-freedom. (???) |

## video Command

This command is used to construct a TclVideoPlayer object for displaying the images in a file created by the recorder display (page 91) command.

video -file $fileName -window $windowName

| $fileName | fileName of images file created by the recorder display (page 91) command |
| --- | --- |
| $windowName | name of window to be created |

The images are displayed by invoking the play (page 136) command.

### play Command

This command is used to play the TclVideoPlayer object created by the video (page 136) command.

play

In this chapter, some examples on how to generate input for OpenSees for specific tasks will be presented.

Chapter 27

# How To....

- In This Chapter ...Run OpenSees 138 ...Define Units & Constants 140 ...Generate Matlab Commands 141 ...Define Tcl Procedure 141 ...Read External files 143 Building The Model 144 Defining Output 150 Gravity Loads 152 Static Analysis 153 Dynamic Analysis 155 ...Combine Input-File Components 156 ...Run Parameter Study 157 ...Run Moment-Curvature Analysis on Section 159 ...Determine Natural Period & Frequency 161

## ...Run OpenSees

There are three ways that OpenSees/Tcl commands can be executed:

- Interactive

Commands can be input directly at the propt, as shown in the figure (Win32 version):

- Figure 37: Run OpenSees -- Interactive

- Execute Input File at OpenSees prompt

This method is the most-commonly used one. An external file containing the input commands can be generated a-priori (inputFile.tcl) and be executed at the OpenSees prompt by using the source command. The generation of the input script files is presented in this chapter. The file execution is shown in the figure (Win32 version):

Batch Mode

- Figure 38: Run OpenSees -- Source File

The previously-created input file containing the Tcl script commands necessary to execute the analsis can also be executed at the MS-DOS/Unix prompt, as shown in the figure (Win32 version):

- Figure 39: Run OpenSees -- Batch Mode

## ...Define Units & Constants

The OpenSees interpreter does not process units. Units, however, can be used when entering values if these units are defined previously. The unit definition consists of two parts: the basic units are defined first, all other units are subsequently defined. The basic units are assigned a value of one and all OpenSees output is in these units. It is very important that all basic units are independent of each other. The unit-definition file can contain both metric and Imperial units, as can the basic units. Hence, the input files may contain mixed units.

Constants, such as and g can also be defined apriori.

An example of unit and constant definition is given in the following:

# ----Units&Constants.tcl---------------------------------------------------------

set in 1.;			# define basic units

set sec 1.;

set kip 1.;

set ksi [expr $kip/pow($in,2)];		# define dependent units

set psi [expr $ksi/1000.];

set ft [expr 12.*$in];

set lb [expr $kip/1000];

set pcf  [expr $lb/pow($ft,3)];

set ksi  [expr $kip/pow($in,2)];

set psi  [expr $ksi/1000.];

set cm  [expr $in/2.54];		# define metric units

set meter [expr 100.*$cm];

set MPa [expr 145*$psi];

set PI [expr 2*asin(1.0)];		# define constants

set g [expr 32.2*$ft/pow($sec,2)];

set U 	1.e10;		# a really large number

set u 	[expr 1/$U];	# a really small number

...Generate Matlab Commands

Chapter 28

Matlab is a common tool for post-processing. Matlab command files can be generated using the Tcl scripting language. Using this technique ensures that the same analysis parameters are used.

Here is an example.

# -------------MatlabOutput.tcl-------------------------------------------------------------------

set Xframe 1;		# this parameter would be passed in

set fDir "Data/";

file mkdir $fDir;   		# create directory

set outFileID [open $fDir/DataFrame$Xframe.m w];   # Open output file for writing

puts $outFileID   "Xframe($Xframe) = $Xframe;";	# frame ID

puts $outFileID   "Hcol($Xframe) = $Hcol;";		# column diameter

puts $outFileID   "Lcol($Xframe) = $Lcol;";		# column length

puts $outFileID   "Lbeam($Xframe) = $Lbeam;";		# beam length

puts $outFileID    "Hbeam($Xframe) = $Hbeam;";	# beam depth

puts $outFileID    "Bbeam($Xframe) = $Bbeam;"; 	# beam width

puts $outFileID   "Weight($Xframe) = $Weight;"	;	# superstructure weight

close $outFileID

## ...Define Tcl Procedure

The procedure is a useful tool available from the Tcl language. A procedure is a generalized funcion/subroutine using arguments. Whenever the Tcl procedure is invoked, the contents of body will be executed by the Tcl interpreter.

An example of a Tcl procedure is found in RCcircSec.tcl. It defines a procedure which generates a circular reinforced concrete section with one layer of steel evenly distributed around the perimeter and a confined core:

# ----RCcircSec.tcl------------------------------------------------------------------------

#			by Michael H. Scott

# Define a procedure which generates a circular reinforced concrete section

# with one layer of steel evenly distributed around the perimeter and a confined core.

# Formal arguments

#    id - tag for the section that is generated by this procedure

#    ri - inner radius of the section

#    ro - overall (outer) radius of the section

#    cover - cover thickness

#    coreID - material tag for the core patch

#    coverID - material tag for the cover patches

#    steelID - material tag for the reinforcing steel

#    numBars - number of reinforcing bars around the section perimeter

#    barArea - cross-sectional area of each reinforcing bar

#    nfCoreR - number of radial divisions in the core (number of "rings")

#    nfCoreT - number of theta divisions in the core (number of "wedges")

#    nfCoverR - number of radial divisions in the cover

#    nfCoverT - number of theta divisions in the cover

# Notes

#    The center of the reinforcing bars are placed at the inner radius

#    The core concrete ends at the inner radius (same as reinforcing bars)

#    The reinforcing bars are all the same size

#    The center of the section is at (0,0) in the local axis system

#    Zero degrees is along section y-axis

proc RCcircSection {id ri ro cover coreID coverID steelID numBars barArea nfCoreR nfCoreT nfCoverR nfCoverT} {

section fiberSec $id {

set rc [expr $ro-$cover];		# Core radius

patch circ $coreID $nfCoreT $nfCoreR 0 0 $ri $rc 0 360; 	# Core patch

patch circ $coverID $nfCoverT $nfCoverR 0 0 $rc $ro 0 360;   # Cover patch

if {$numBars <= 0} {

return

}

set theta [expr 360.0/$numBars];		# Angle between bars

layer circ $steelID $numBars $barArea 0 0 $rc $theta 360;	# Reinforcing layer

}

}

This procedure is invoked by the following commands, assuming that all arguments have been defined previously in the input generation:

source RCcircSection.tcl;

RCcircSection $IDcolFlex  $riCol $roCol $cover $IDcore $IDcover $IDsteel $NbCol  $AbCol  $nfCoreR $nfCoreT $nfCoverR $nfCoverT

NOTE: the file containing the definition of the procedure (RCcircSec.tcl) needs to be sourced before the procedure is invoked.

## ...Read External files

External files may either contain Tcl commands or data.

- Common input file

The external file may contain a series of commands that is common in most analyses. One set of Tcl commands that can be stored in a external file are ones which define units.

An example of an external file that may want to be read within the input commands is the unit-definition file presented earlier (units&constants.tcl).

This file is invoked with the following command:

source units.tcl

- Repeated Calculations

An external file may contain a series of calculations that are repeated. An example of this is a parameter study:

set Hcolumn 66;

source analysis.tcl

set Hcolumn 78;

source analysis.tcl

The analysis.tcl file contains the commands that set up and execute the entire analysis.

- External Data File

The following commands open a data file (filename=inFilename), read the file row by row and assign the value of each row to the a single variable (Xvalues). If there are more than one value in the row, $Xvalues is a list array, and the individual components may be extracted using the lindex command. The user may change the commands to be exectued once the data-line has been read to whatever is needed in the analysis.

# ----ReadData.tcl--------------------------------------------------------------------

if [catch {open $inFilename r} inFileID] {;	# Open the input file and check for error

puts stderr "Cannot open $inFilename for reading";	# output error statement

} else {

foreach line [split [read $inFileID] \n] { ;	# Look at each line in the file

if {[llength $line] == 0} {;	# Blank line --> do nothing

continue;

} else {

set Xvalues $line;	# execute operation on read data

}

}

close $inFileID;   ;	# Close the input file

}

## Building The Model

### ...Define Variables and Parameters

In the Tcl scripting language variables may be used to represent numbers. Once defined, these variables can be use instead of numbers in Tcl and OpenSees commands. When they are being recalled, the variables are precedented by the symbol $. If this symbol is not used, the variable name is interpreted as a string command and an error may result.

A few examples are given:

- MATERIAL PARAMETERS:

# -----MaterialParameters.tcl-------------------------------------------------------

set fc [expr -4.0*$ksi];		#  nominal compressive strength of concrete

set Ec [expr 57*$ksi*sqrt(-$fc/$psi)];	# Concrete Elastic Modulus

set fc1C [expr 1.26394*$fc];		# CONFINED concrete (mander model), max stress

set eps1C [expr 2.*$fc1C/$Ec];	# strain at maximum stress

set fc2C $fc;			# ultimate stress

set eps2C [expr 2.*$fc2C/$Ec];	# strain at ultimate stress

set fc1U $fc;			# UNCONFINED concrete (parabolic model), max stress

set eps1U -0.003;		# strain at maximum stress

set fc2U [expr 0.1*$fc];		# ultimate stress

set eps2U -0.006;		# strain at ultimate stress

set Fy [expr 70.*$ksi];		# STEEL yield stress

set Es [expr 29000.*$ksi];	# elastic modulus of steel

set epsY [expr $Fy/$Es];		# steel yield strain

set Fy1 [expr 95.*$ksi];		# steel stress post-yield

set epsY1 0.03;			# steel strain post-yield

set Fu [expr 112.*$ksi];		# ultimate stress of steel

set epsU 0.08;  		# ultimate strain of steel

set Bs [expr ($Fu-$Fy)/($epsU-$epsY)/$Es];   # post-yield stiffness ratio of steel

set pinchX 	1.0;		# pinching parameter for hysteretic model

set pinchY 	1.0;		# pinching parameter for hysteretic model

set damage1 	0.0;		# damage parameter for hysteretic model

set damage2 	0.0;		# damage parameter for hysteretic model

set betaMUsteel 0.0;		# degraded unloading stiffness for hysteretic material based on MU^(-beta)

set betaMUjoint 0.0;		# degraded unloading stiffness for hysteretic material based on MU^(-beta) -- timoshenko value of 0.5

set betaMUph 	0.0;		# degraded unloading stiffness for hysteretic material based on MU^(-beta) -- timoshenko value of 0.5

set G 	$U;		# Torsional stiffness Modulus

set J 	1.;		# Torsional stiffness of section, place here just to keep with G

set GJ [expr $G*$J];	# Torsional stiffness

- ELEMENT PARAMETERS:

# -----ElementParameters.tcl----------------------------------------------------

set Hcol   	[expr 5.*$ft];	# column diameter

set Lcol   	[expr 36*$ft];	# column length

set Hbeam   	[expr 8.*$ft];	# beam depth

set Lbeam 	[expr 36.*$ft];	# beam length

set GrhoCol 	0.0125;	# column longitudinal-steel ratio

set Weight	[expr 2000.*$kip];	# superstructure weight

set Bbeam 	$Hcol;	# beam width

set Rcol 	[expr $Hcol/2];	# COLUMN radius

set Acol 	[expr $PI*pow($Rcol,2)];	# column cross-sectional area

set cover 	[expr $Hcol/15];	# column cover width

set IgCol 	[expr $PI*pow($Rcol,4)/4];	 # column gross moment of inertia, uncracked

set IyCol 	$IgCol;	# elastic-column properties

set IzCol 	$IgCol;	# elastic-column properties

set IzBeam	[expr $Bbeam*pow($Hbeam,3)/12]; # beam gross moment of inertia, about horizontal Z-axis

set IyBeam 	[expr $Hbeam*pow($Bbeam,3)/12]; # beam gross moment of inertia, about the vertical Y-axis

set Abeam 	[expr $Hbeam*$Bbeam*10000]; # beam cross-sectional area, make it very very stiff

# define COLUMN section parameters

set NbCol	20;	# number of column longitudinal-reinforcement bars

set AsCol	[expr $GrhoCol*$Acol];	# total steel area in column section

set AbCol	[expr $AsCol/$NbCol];	# bar area of column longitudinal reinforcement

set riCol 	0.0;	# inner radius of column section

set roCol 	$Rcol;	# outer radius of column section

set IDcore 	1;	# ID tag for core concrete

set IDcover 	2;	# ID tag for cover concrete

set IDsteel 	3;	# ID tag for steel

set nfCoreR	8;	# number of radial fibers in core

set nfCoreT 	16;	# number of tangential fibers in core

set nfCoverR 	 2;	# number of radial fibers in cover

set nfCoverT 	16;	# number of tangential fibers in cover

set IDcolFlex 	2;	# ID tag for column section in flexure, before aggragating torsion

set IDcolTors 	10;	# ID tag for column section in torsion

set IDcolSec 	1;	# ID tag for column section

set IDcolTrans 	1;	# ID tag for column transformation, defining element normal

set IDbeamTrans	2;	# ID tag for beam transformation, defining element normal

set np 	5;	 # Number of integration points

- GRAVITY PARAMETERS:

# -----GravityParameters.tcl----------------------------------------------------

# define GRAVITY paramters

set Pdl	[expr $Weight/2];		# gravity axial load per column

set Wbeam 	[expr $Weight/$Lbeam];		# gravity dead load distributed along beam length

set Mdl 	[expr $Wbeam*pow($Lbeam,2)/12];	# nodal moment due to distributed dl

set Mass 	[expr $Weight/$g];			# mass of superstructure

set Mnode 	[expr $Mass/2];			# nodal mass for each column joint

- ANALYSIS PARAMETERS:

# -----AnalysisParameters.tcl----------------------------------------------------

set DxPush 	[expr 0.1*$in];	# Displacement increment for pushover analysis

set DmaxPush	[expr 0.05*$Lcol];	# maximum displamcement for pushover analysis

set DtAnalysis	[expr 0.005*$sec];# time-step Dt for lateral analysis

set DtGround 	[expr 0.02*$sec];	# time-step Dt for input grond motion

set TmaxGround [expr 35 *$sec];	# maximum duration of ground-motion analysis

set gamma 	0.5;		# gamma value for newmark integration

set beta 	0.25		# beta value for newmark integration

### ...Build Model and Define Nodes

This example shows how to set up the geometry of the structure shown in the Figure (page 148).

These commands are typically placed at the beginning of the input file, after the header remarks.

# construct model builder using the model Command (page 17)

wipe;			#  clear data from past analysis

model basic -ndm 3 -ndf 6;	# modelbuilder: basic (page 17), ndm= no. dimensions, ndf= no.

# Define nodes  ------ frame is in X-Y plane (X-horizontal, Y-vertical) using the node Command (page 18)

node  1     0.        0.      0.;	# base of left column

node  2     360.     0.      0.;	# base of right columnn

node  3     0.        120.   0.;	# top of left column

node  4     360.     120.   0.;	# top of right columnn

# Define Boundary Conditions and nodal mass using the fix Command (page 20)  ! 1: restrained, 0: released

fix   1   1  1  1  1  1  1;

fix   2   1  1  1  1  1  1;

fix   3   0  1  1  1  1  0    -mass [expr 2000/2/386.4] 0.  0.  0.  0.  0.

fix   4   0  1  1  1  1  0

# define mass at node 4 using the mass Command (page 19):

mass 4  [expr 2000/2/32.2/12] 0.  0.  0.  0.  0.

# NOTE: The second command assigning the mass of a specific node will override any previous mass assignments to that node.

### ...Build Model and Define Nodes using Variables

This example sets up the geometry of the cantilever column shown in the Figure (page 148) -- using variables.

# -------Build&Nodes.tcl-------------------------------------------------------

wipe;			#  clear data from past analysis

model basic -ndm 3 -ndf 6;

# define units and constants

# source units.tcl; 	# if contained in external file

# Define nodes  ------ frame is in X-Y plane (X-horizontal, Y-vertical) using the node Command (page 18)

node  1     0.               0.         0.;	# base of left column

node  2     $Lbeam     0.         0.;	# base of right columnn

node  3     0.               $Lcol    0.;	# top of left column

node  4     $Lbeam     $Lcol    0.;	# top of right columnn

# Define Boundary conditions using the fix Command (page 20)  ! 1: restrained, 0: released

fix   1   1  1  1  1  1  1;	# fully-fixed support

fix   2   1  1  1  1  1  1;

fix   3   0  1  1  1  1  0    -mass $Mass 0.  0.  0.  0.  0.;

fix   4   0  1  1  1  1  0;

# define the mass at node 4 using the mass Command (page 19):

mass 4 $Mass 0.  0.  0.  0.  0.;

# NOTE: The second command assigning the mass of a specific node will override any previous mass assignements to that node.

### ...Define Materials

The following is an example on how to define materials for reinforced-concrete structures. The examples assume that the variables have been defined apriori. If these commands are placed into an external file they can be used in a number of analyses without significant modifications using the source command.

# -------MaterialsRC.tcl----------------------------------------------------------

set ConcreteMaterialType "inelastic";	# options: "elastic","inelastic"

set SteelMaterialType "hysteretic";		# options: "elastic","bilinear","hysteretic"

# CONCRETE

if {$ConcreteMaterialType =="elastic"} {

uniaxialMaterial Elastic $IDcore $Ec

uniaxialMaterial Elastic $IDcover $Ec

}

if {$ConcreteMaterialType == "inelastic"} {

uniaxialMaterial Concrete01 $IDcore $fc1C $eps1C $fc2C $eps2C;	# Core concrete

uniaxialMaterial Concrete01 $IDcover $fc1U $eps1U $fc2U $eps2U;	# Cover concrete

}

# STEEL

if {$SteelMaterialType == "elastic"} {

uniaxialMaterial Elastic $IDsteel $Es

}

if {$SteelMaterialType == "bilinear"} {

uniaxialMaterial Steel01     $IDsteel      $Fy      $Es      $Bs

}

if {$SteelMaterialType == "hysteretic"} {

uniaxialMaterial Hysteretic $IDsteel $Fy $epsY $Fy1 $epsY1 $Fu $epsU -$Fy -$epsY -$Fy1 -$epsY1 -$Fu -$epsU $pinchX $pinchY $damage1 $damage2 $betaMUsteel

}

### ...Define Elements

# -----ELEMENTS.tcl-------------------------------------------------------

# COLUMNS

set ColumnType "inelastic";	# options: "rigid" "elastic" "inelastic"

set np 5;			# number of integration points

# source RCcircSection.tcl;	# proc to define circular fiber section for flexural characteristics

RCcircSection $IDcolFlex  $riCol $roCol $cover $IDcore $IDcover $IDsteel $NbCol  $AbCol  $nfCoreR $nfCoreT $nfCoverR $nfCoverT

uniaxialMaterial Elastic $IDcolTors $GJ;		# Define torsional stiffness

section Aggregator $IDcolSec    $IDcolTors      T    -section    $IDcolFlex;	# attach torsion and flexure

geomTransf Linear      $IDcolTrans            0 0 1;		# Linear: no second-order effects

if {$ColumnType == "rigid"} {

set $IyCol [expr $IyCol*$IyCol];

set $IzCol [expr $IzCol*$IzCol];

element elasticBeamColumn   1   1  3 $Acol    $Ec   $G   $J   $IyCol   $IzCol   $IDcolTrans

element elasticBeamColumn   2   2  4 $Acol    $Ec   $G   $J   $IyCol   $IzCol   $IDcolTrans

}

if {$ColumnType == "elastic"} {

element elasticBeamColumn   1   1  3 $Acol    $Ec   $G   $J   $IyCol   $IzCol   $IDcolTrans

element elasticBeamColumn   2   2  4 $Acol    $Ec   $G   $J   $IyCol   $IzCol   $IDcolTrans

}

if {$ColumnType == "inelastic"} {

element nonlinearBeamColumn  1   1  3    $np $IDcolSec  $IDcolTrans

element nonlinearBeamColumn  2   2  4    $np $IDcolSec  $IDcolTrans

}

# BEAM

geomTransf Linear    $IDbeamTrans       0 0 1;	# define orientation of beam local axes

element elasticBeamColumn   3  3 4   $Abeam    $Ec   $G $J   $IyBeam    $IzBeam     $IDbeamTrans;

## Defining Output

### ...Define Analysis-Output Generation

Different output may be generated depending on whether the analysis is static or dynamic. Here is an example:

# -------Output.tcl-------------------------------------------------------------------------

set fDir "Data/";

file mkdir $fDir;   		# create directory

set ANALYSIS "Static";   # this variable would be passed in

set IDctrlNode 3;     # this variable would be passed in

if {$ANALYSIS == "Static"} {

# Record nodal displacements -NODAL DISPLACEMENTS

set fDstatFrame DStatFrame[expr $Xframe]

set iNode "$IDctrlNode";	    # can add node numbers to this list

foreach xNode $iNode {

set fNode Node$xNode

set Filename $fDir$fDstatFrame$fNode

recorder Node $Filename.out disp -time -node $xNode -dof 1 6;

};   # end of xNode

# Record element forces and deformations - COLUMNS

set iEL "1 2"

set fFstatFrame FStatFrame[expr $Xframe]

set fDstatFrame DStatFrame[expr $Xframe]

foreach xEL $iEL {

set fEl El[expr $xEL]

set iSEC "1 3 5"

set Ffilename $fDir$fFstatFrame$fEl

recorder Element $xEL -time -file $Ffilename.out localForce

foreach xSEC $iSEC {

set fSec Sec[expr $xSEC]

set Dfilename $fDir$fDstatFrame$fEl$fSec

recorder Element $xEL -file $Dfilename.out -time section $xSEC deformations

};	# end of xSEC

};		# end of xEL

};		# end of static analysis

set ANALYSIS "Dynamic";   # this variable would be passed in

set GroundFile "ElCentro";     # this variable would be passed in

if {$ANALYSIS == "Dynamic"} {

set fDDynaFrame DDynaFrame[expr $Xframe]

set fGroundFile $GroundFile

set Filename $fDir$fDDynaFrame$fGroundFile

# Record nodal displacements

recorder Node $Filename.out disp -time -node $IDctrlNode -dof 1

};     # end of dynamic analysis

### ...Define Data-Plot During Analysis

# ---------RecorderPlot.tcl------------------------------------------------------------------------------------

set pfile "Data/node.out";

recorder Node $pfile disp -time -node $IDctrlNode -dof 1

set title PushFrame$Xframe;

recorder plot $pfile $title  0 0 350 350 -columns 2 1

set pfile "Data/Elem1.out";

set title PushElem1;

recorder Element 1 -time -file $pfile globalForce

recorder plot $pfile $title 400 0 350 350 -columns 2 1

## Gravity Loads

### ...Define Gravity Loads

# ---------DefineGravity.tcl--------------------------------------------------------------------------------------

set GravSteps 10

pattern Plain 1 Linear {

load 3	0. -$Pdl   0.   0.   0.   -$Mdl ;   # Fx   Fy  Fz  Mx  My  Mz

load 4	0. -$Pdl   0.   0.   0.   +$Mdl

}

system UmfPack;	# solution procedure, how it solves system of equations

constraints Plain;	# how it handles boundary conditions, enforce constraints

test NormDispIncr 1.0e-5    10      0;

algorithm Newton;

numberer RCM;				# renumber dof's to minimize band-width

integrator LoadControl [expr 1./$GravSteps] 1 [expr 1./$GravSteps] [expr 1./$GravSteps]

analysis Static

initialize; # this command will not be necessary in new versions of OpenSees

### ...Run Gravity Analysis

# ---------RunGravity.tcl------------------------------------------------------------------------------------

analyze $GravSteps		# run gravity analysis

loadConst -time 0.0;		# keep gravity load and restart time -- lead to lateral-load analysis

## Static Analysis

### ...Define Static Pushover Analysis

The following commands are executed once the gravity loads have been defined and applied

# ---------DefinePushover.tcl--------------------------------------------------------------------------------------------------

set analysis "STATIC";	# this variable would be passed in

# the following settings do not need to be here if they have been defined in the gravity analysis

system UmfPack;

constraints Plain;

test NormDispIncr 1.0e-5    10      0;

algorithm Newton;

numberer RCM;	analysis Static;

# ------------

set PUSHOVER "DispControl";	# run displacement-controlled static pushover analysis

pattern Plain 2 Linear {

load $IDctrlNode 	100.0 0.0 0.0 0.0 0.0 0.0

}

if {$PUSHOVER == "LoadControl"} {

integrator LoadControl  0.2 4  0.1 2.0

set Nsteps 20

} elseif {$PUSHOVER == "DispControl"} {

integrator DisplacementControl  $IDctrlNode   1   $DxPush 1 $DxPush $DxPush

set Nsteps [expr int($DmaxPush/$DxPush)]

} else {

puts stderr "Invalid PUSHOVER option"

}

### ...Run Static Pushover Analysis

While running a static pushover analysis may take a single command, convergence may not always be reached with a single analysis-parameter setting. A Tcl script which tries different solutions can be incorporated to improve the chances of convergence.

- No convergence issues

The following command executes the static push-over analysis when convergence is not a problem.

# ---------RunPushover.tcl------------------------------------------------------------------------------------

analyze $Nsteps

- Convergence attemps

The following Tcl script should be incorporated in the input file to run a number of attempts at convergence:

# ---------RunPushover2Converge.tcl----------------------------------------------------------------

set ok [analyze $Nsteps]

# if analysis fails, try the following, performance is slowed inside this loop

if {$ok != 0} {

set ok 0;

set maxU $DmaxPush

set controlDisp 0.0;

test NormDispIncr   1.0e-8          20        0

while {$controlDisp < $maxU && $ok == 0} {

set ok [analyze 1]

set controlDisp [nodeDisp $IDctrlNode 1]

if {$ok != 0} {

puts "Trying Newton with Initial Tangent .."

test NormDispIncr   1.0e-8          1000  1

algorithm Newton -initial

set ok [analyze 1]

test NormDispIncr   1.0e-8          20        0

algorithm Newton

}

if {$ok != 0} {

puts "Trying Broyden .."

algorithm Broyden 8

set ok [analyze 1]

algorithm Newton

}

if {$ok != 0} {

puts "Trying NewtonWithLineSearch .."

algorithm NewtonLineSearch .8

set ok [analyze 1]

algorithm Newton

}

};	# end while loop

};	# end original if $ok!=0 loop

if {$ok != 0} {

puts "DispControl Analysis FAILED"

puts "Do you wish to continue y/n ?";	# include if want to pause at analysis failure

gets stdin ans;		# not recommended in parameter study

if {$ans == "n"} done;		# as it interrupts batch file

} else {

puts "DispControl Analysis SUCCESSFUL"

}

## Dynamic Analysis

### ...Define Dynamic Ground-Motion Analysis

# ---------DefineDynamic.tcl----------------------------------------------------------------

wipeAnalysis

system UmfPack

constraints Plain

test NormDispIncr 1.0e-8 20 0;

algorithm Newton

numberer RCM

integrator Newmark $gamma $beta   $alphaM $betaK $betaKcomm $betaKinit;

analysis Transient

set Nsteps [expr int($TmaxGround/$DtAnalysis)];

# read a PEER strong motion database file, extracts dt from the header and converts the file

# to the format OpenSees expects for uniform ground motions

source ReadSMDFile.tcl;

set dir "GMfiles/"

set outFile $dir$GroundFile.g3;	# set variable holding new filename

set inFile $dir$GroundFile.th

ReadSMDFile $inFile $outFile dt;	# convert the ground-motion file

set GMfatt [expr $g*$GMfact];		# data in input file is factor of g

set Gaccel "Series -dt $dt -filePath $outFile -factor  $GMfatt";	# time series information

pattern UniformExcitation  2   1  -accel $Gaccel;		# create uniform excitation

### ...Run Dynamic Ground-Motion Analysis

# ---------RunDynamicGM.tcl------------------------------------------------------------

analyze $Nsteps $DtAnalysis;

## ...Combine Input-File Components

A series of Tcl-script input-file components have been presented in this section. These components can be combined to perform a static lateral-load analysis of the portal frame under consideration using the source command:

# ----FullStaticAnalysis.tcl---------------------------------------------------------

wipe

model basic -ndm 3 -ndf 6

source Units&Constants.tcl

source MaterialParameters.tcl

source ElementParameters.tcl

source GravityParameters.tcl

source AnalysisParameters.tcl

source MatlabOutput.tcl

source BuildModel&Nodesw/Variables--portal.tcl

source materialsRC.tcl

source RCcircSec.tcl

source Elements.tcl

source Output.tcl

source DefineGravity.tcl

source runGravity.tcl

source DefinePushover.tcl

source RunPushover2Converge.tcl

This method of breaking the input file into components is convenient when the size of the problem does not permit manageability of a single input file.

## ...Run Parameter Study

In a parameter study, the same analysis is performed on a number of models where only certain properties are varied, such as column height. There are two common types of parameter studies shown in this section: series and parallel parsing.

The following diagram illustrates the difference between series and parallel parsing for two parameter lists [1 2 3] and [A B C]:

- Parsing in series

In this type of study, one parameter is held constant, while the others are parsed in sequence:

# -----ParameterStudySeries.tcl----------------------------------------------------

source units.tcl

set iHcol "[expr 5.*$ft] [expr 6.5*$ft]";	# column diameter

set iLcol "[expr 36*$ft] [expr 42*$ft]";	# column length

set Xframe 0;		# initialize Frame Counter, used in output

foreach Hcol $iHcol {

foreach Lcol $iLcol {

set Xframe [expr $Xframe+1];

set ANALYSIS "Static";

source Analysis.tcl*

};   # close iLcol loop

};   # close iHcol loop

*NOTE: The file Analysis.tcl contains all the model and analysis commands.

- Parsing in parallel

In this study, the ith elements of each parameter list are considered together, resulting in fewer study models.

# -----ParameterStudyParallel.tcl----------------------------------------------------

source units.tcl

set iHcol "[expr 5.*$ft] [expr 6.5*$ft]";	# column diameter

set iLcol "[expr 36*$ft] [expr 42*$ft]";	# column length

set Xframe 0;		# initialize Frame Counter, used in output

foreach Hcol $iHcol Lcol $iLcol{

set Xframe [expr $Xframe+1];

set ANALYSIS "Static";

source Analysis.tcl*

};   # close iHcol & iLcol loop

*NOTE: The file Analysis.tcl contains all the model and analysis commands.

## ...Run Moment-Curvature Analysis on Section

A procedure for performing section analysis (only does moment-curvature, but can be easily modified to do any mode of section reponse):

# ---------MPhiProc.tcl-------------------------------------------------------------------------------

# Sets up a recorder which writes moment-curvature results to file

#   make sure the node and element numbers are not used elsewhere in the model

# Arguments

#	secTag -- tag identifying section to be analyzed

#	axialLoad -- axial load applied to section (negative is compression)

#	maxK -- maximum curvature reached during analysis

#	numIncr -- number of increments used to reach maxK (default 100)

proc MomentCurvature {secTag axialLoad maxK {numIncr 100} } {

node 1001 0.0 0.0 0.0;   # Define two nodes at (0,0)

node 1002 0.0 0.0 0.0

fix 1001 1 1 1 1 1 1; # Fix all degrees of freedom except axial and bending

fix 1002 0 1 1 1 1 0

element zeroLengthSection  2001   1001   1002  $secTag

recorder Node Mphi.out disp -time -node 1002 -dof 6;# output moment & curvature

integrator LoadControl 0 1 0 0;   # Define analysis parameters

system SparseGeneral -piv;	# Overkill, but may need the pivoting!

test NormUnbalance 1.0e-9 10

numberer Plain;

constraints Plain;

algorithm Newton;

analysis Static;

pattern Plain 3001 "Constant" {

load 1002 $axialLoad 0.0 0.0 0.0 0.0 0.0

};   # Define constant axial load

analyze 1;  # Do one analysis for constant axial load

pattern Plain 3002 "Linear" {

load 1002 0.0 0.0 0.0 0.0 0.0 1.0

};   # Define reference moment

set dK [expr $maxK/$numIncr]; # Compute curvature increment

# Use displacement control at node 1002 for section analysis, dof 6

integrator DisplacementControl 1002 6 $dK 1 $dK $dK

analyze $numIncr; 	# Do the section analysis

}

When including this procedure, ensure that the node and element numbers used by it are not used elsewhere in the OS model.

The above procedure may be incorporated into the static pushover analysis file:

# ----MomentCurvature.tcl---------------------------------------------------------

wipe

model basic -ndm 3 -ndf 6

source Units&Constants.tcl

source MaterialParameters.tcl

source ElementParameters.tcl

source GravityParameters.tcl

source materialsRC.tcl

source RCcircSec.tcl

RCcircSection $IDcolSec  $riCol $roCol $cover $IDcore $IDcover $IDsteel $NbCol  $AbCol  $nfCoreR $nfCoreT $nfCoverR $nfCoverT

source MPhiProc.tcl

set phiYest [expr $epsY/(0.7*$Hcol)];	# estimate yield curvature

set axialLoad -$Pdl;			# define axial load -- +tension in Mom-curv analysis

set maxK [expr 20*$phiYest];		# maximum curvature reached during analysis

MomentCurvature $IDcolSec $axialLoad $maxK;

## ...Determine Natural Period & Frequency

The natural period and frequency of the structure can be determined at any point during the analysis using the eigen (page 130) command. In turn, these quantities can be stored as variables and used in defining analysis parameters, such as rayleigh-damping parameters:

# ------------PeriodFreq&Damping.tcl-----------------------------------------------------------------

# determine Natural Period, Frequency & damping parameters for SDOF

set $xDamp 0.02;	# damping ratio (0.02-0.05-typical)

set lambda [eigen 1]

set omega [expr pow($lambda,0.5)]

set Tperiod  	[expr 2*$PI/$omega];	   # period (sec.)

puts $Tperiod

set alphaM 	0;		# stiffness-prop. RAYLEIGH damping parameter; D = alphaM*M

set betaK	 	0;	# stiffness proportional damping;		+beatK*KCurrent

set betaKcomm  	[expr 2*$xDamp/$omega];	# mass-prop. RAYLEIGH damping parameter;	+betaKcomm*KlastCommitt

set betaKinit 	0;		# initial-stiffness proportional damping		+beatKinit*Kini

.

# Index

...Build Model and Define Nodes • 19, 20, 21, 22, 157

...Build Model and Define Nodes using Variables • 158

...Combine Input-File Components • 166

...Define Analysis-Output Generation • 161

...Define Data-Plot During Analysis • 162

...Define Dynamic Ground-Motion Analysis • 165

...Define Elements • 160

...Define Gravity Loads • 162

...Define Materials • 159

...Define Static Pushover Analysis • 163

...Define Tcl Procedure • 152

...Define Units & Constants • 151

...Define Variables and Parameters • 155

...Determine Natural Period & Frequency • 170

...Generate Matlab Commands • 152

...Read External files • 154

...Run Dynamic Ground-Motion Analysis • 166

...Run Gravity Analysis • 163

...Run Moment-Curvature Analysis on Section • 168

...Run OpenSees • 148

...Run Parameter Study • 167

...Run Static Pushover Analysis • 164

A

algorithm Command • 17, 116, 117, 118, 128, 131, 144

analysis Command • 17, 116, 124, 125, 139, 144

Analysis Commands • 104

Analysis Object • 16, 17, 116

analyze Command • 139

Arc-Length Control • 117, 118, 123, 125

B

BandGeneral SOE • 137

BandSPD SOE • 137

Basic Model Builder • 8, 19, 20, 21, 56, 57, 58, 60, 61, 96, 97, 102, 157

Bbar Brick Element • 45, 79, 88

Bbar Plane Strain Quadrilateral Element • 45, 77, 86

Beam With Hinges Element • 71, 72, 99, 101

BFGS Algorithm • 130

Bidirectional Section • 66

block Command • 15, 20, 85

block2D Command • 85, 86

block3D Command • 85, 87

Bond01 Material • 43

Bond02 Material • 44

Brick Elements • 78

Broyden Algorithm • 130

build Command • 20

Building The Model • 155

C

Circular Layer Command • 54, 61

Circular Patch Command • 58

Concrete01 Material • 36, 40

Concrete02 Material • 40

Concrete03 Material • 42

Constant Time Series • 106

constraints Command • 15, 17, 20, 110, 116, 117, 118, 119, 144

Constraints Commands • 22

Copyright • 9

Corotational Transformation • 93

Corotational Truss Element • 69

D

dataBase Commands • 141

Defining Output • 161

Displacement Control • 116, 123, 124

Displacement-Based Beam-Column Element • 71, 73, 99, 101

Display Recorder • 101, 146

Domain Object • 16, 100, 110, 144

Drift Recorder • 98

Dynamic Analysis • 165

E

eigen Command • 140, 170

Eight Node Brick Element • 80

Elastic Beam Column Element • 70, 99, 101

Elastic Isotropic Material • 45

Elastic Material • 27

Elastic Membrane Plate Section • 66

Elastic Section • 52

Elastic-No Tension Material • 37

Elastic-Perfectly Plastic Gap Material • 30

Elastic-Perfectly Plastic Material • 28

eleLoad Command • 110, 112

element Command • 15, 20, 68, 99, 100, 144

Element Recorder • 27, 45, 51, 69, 70, 72, 73, 74, 75, 76, 77, 78, 81, 82, 83, 98

Energy Increment Test • 133

Enhanced Strain Quadrilateral Element • 45, 77, 86

EnvelopeElement Recorder • 100

EnvelopeNode Recorder • 96

EPState • 47, 50

equalDOF Command • 25

Evolution Law • 47, 49

Example Tcl Commands • 12

F

Fedeas Materials • 40

Fiber Command • 54, 55

Fiber Section • 54

FileDatastore Command • 141

Fix Command • 22, 158, 159

fixX Command • 23

fixY Command • 23

fixZ Command • 24

G

Geometric Transformation Command • 15, 20, 70, 71, 73, 90

getTime Command • 145

Gravity Loads • 162

groundMotion Command • 105, 106, 113, 114, 115

H

Hardening Material • 34

Hilbert-Hughes-Taylor • 117, 118, 123, 127

How To.... • 147

Hysteretic Material • 38

I

imposedMotion Command • 113, 115

integrator Command • 17, 114, 116, 117, 118, 122

Interpolated GroundMotion • 114

Introduction • 7

J

J2 Plasticity Material • 45

K

Krylov-Newton Algorithm • 130

L

Lagrange Multipliers • 121

Linear Algorithm • 128

Linear Time Series • 106, 123

Linear Transformation • 90, 92, 93

load Command • 110, 111

Load Control • 116, 117, 123

loadConst Command • 145

M

Mass Command • 15, 20, 21, 158, 159

MaxNodeDisp Recorder • 97

Minimum Unbalanced Displacement Norm • 116, 123, 125

Miscellaneous Commands • 143

Model Command • 19, 23, 24, 25, 26, 93, 94, 97, 111, 112, 115, 124, 145, 157

Model Commands • 19

ModelBuilder Object • 15, 16, 20, 22, 23, 70, 72

Modified Newton Algorithm • 129

MultipleSupport Pattern • 113, 114, 115

Multi-Point Constraints • 24

N

nDMaterial Command • 15, 20, 45, 76, 77, 78, 79, 82, 144

Newmark Method • 117, 118, 123, 126

Newton Algorithm • 116, 117, 118, 128

Newton with Line Search Algorithm • 129

Node Command • 8, 15, 20, 98, 144, 158

Node Recorder • 95, 140

nodeDisp Command • 145

Nonlinear Beam Column Element • 71, 99, 101

NonLinear Beam-Column Elements • 52, 71

Norm Displacement Increment T • 132

Norm Unbalance Test • 116, 117, 118, 131

Notation • 7

numberer Command • 17, 116, 117, 118, 134

O

Open System for Earthquake Engineering Simulation • 14

OpenSees Interpreter • 13, 144

P

Parallel Material • 31

Path Time Series • 109

pattern Command • 15, 20, 105, 106, 107, 108, 109, 110, 145

P-Delta Transformation • 92

Penalty Method • 120

Plain Constraints • 116, 117, 118, 120

Plain GroundMotion • 114

Plain Numberer • 134

plain Pattern • 110

Plane Stress Material • 46

Plate Fiber Material • 46, 66

Plate Fiber Section • 66, 76

play Command • 146

playback Command • 102

Plot Recorder • 102

Potential Surface • 47, 48

print Command • 140, 143

ProfileSPD SOE • 116, 117, 118, 137

Q

Quad Element • 45, 75, 86

Quadrilateral Elements • 75

Quadrilateral Patch Command • 54, 56

R

RCM Numberer • 116, 117, 118, 134, 135

recorder Command • 18, 95, 102

Recorder Object • 16, 18

Rectangular Time Series • 107

region Command • 89

reset Command • 144

restore Command • 141, 142

rigidDiaphragm Command • 25

rigidLink Command • 26

S

save Command • 141, 142

Section Aggregator • 62

section Command • 15, 20, 51, 63, 68, 69, 71, 72, 73, 75, 76, 99, 101

Series Material • 32

Shell Element • 45, 76, 86

Sine Time Series • 108

Single-Point Constraints • 22

sp Command • 110, 112

SparseGeneral SOE • 137

SparseSPD SOE • 138

Standard Brick Element • 45, 78, 88

Static Analysis • 17, 116, 123, 163

Steel01 Material • 35, 40

Steel02 Material • 43

Straight Layer Command • 54, 59

system Command • 17, 116, 117, 118, 131, 132, 133, 136, 144

T

Tcl Basics • 9

Tcl Commands • 10, 13

Template Elasto-Plastic Material • 47

test Command • 131

Time Series • 15, 20, 105, 106, 107, 108, 109, 110, 112, 114

Transformation Method • 121

Transient Analysis • 17, 117, 123, 139

Truss Element • 68

Twenty Node Brick Element • 82

U

UmfPack SOE • 137

Uniaxial Section • 53

uniaxialMaterial Command • 15, 20, 27, 31, 32, 53, 54, 56, 58, 59, 61, 62, 63, 68, 69, 74, 144

UniformExcitation Pattern • 112

u-p-U element • 83

V

VariableTransient Analysis • 17, 117, 139

video Command • 146

Viscous Material • 39

W

wipe Command • 144

wipeAnalysis Command • 144

Y

Yield Surface • 47

Z

Zero-Length Element • 74

Zero-Length Elements • 74

Zero-Length Section Element • 75